REPORT FORMAT: V-L5 (Very Large with P&M) | Version: 4.2_2017

File No.: VIS (2021-22)-PL-609-527-664		                     Dated: 16.12.2021

# PROJECT VALUATION REPORT

# OF

# SUB-CRITICAL COAL FIRED

# 600 MW (2x300 MW) THERMAL POWER PLANT

# SITUATED AT

BUTIBORI, MIDC INDUSTRIAL AREA, NAGPUR, MAHARASHTRA, INDIA

OWNER/ PROMOTER

M/S. VIDARBHA INDUSTRIES POWER LIMITED

| REPORT PREPARED FOR | REPORT PREPARED BY |
| --- | --- |
| AXIS BANK LIMITED, STRATEGIC SOLUTIONS GROUP, P.B.MARG, WORLI, MUMBAI, MAHARASHTRA | R.K. ASSOCIATES VALUERS & TECHNO ENGINEERING CONSULTANT PVT. LTD. |

**Important - In case of any query/ issue or escalation you may please contact Incident Manager

at valuers@rkassociates.org. We will appreciate your feedback in order to improve our services.

NOTE: As per IBA Guidelines please provide your feedback on the report within 15 days of its submission after which report will be considered to be correct.

IMPORTANT NOTICE

COPYRIGHT FORMAT: This report is prepared on the copyright format of R.K Associates to serve our clients with the best possible information and analysis to facilitate them to take rational business decisions. Legally no one can copy or distribute this format without prior approval from R.K Associates. It is meant only for the advisory/ reference purpose for the organization/s as mentioned on the cover page of this report. Distribution or use of this format by any organization or individual other than R.K Associates will be seen as an unlawful act and necessary legal action can be taken against the defaulters.

This report is intended for the sole use of the intended recipient/s and contains material that is STRICTLY CONFIDENTIAL AND PRIVATE.

DEFECT LIABILITY PERIOD: - In case of any query/ issue or escalation you may please contact Incident Manager at valuers@rkassociates.org. We ensure 100% accuracy in the calculations done, rates adopted and various other data points & information mentioned in the report but still can’t rule out typing, human errors or any other mistakes. In case you find any such mistake or inaccuracy in any data point of the report, please help us by bringing all such points into our notice immediately or within 15 days of the report delivery in writing, to rectify these timely failing after which R.K Associates will not be responsible for any such inaccuracy in any manner. We will highly appreciate your feedback in order to improve our services.

Part K: R.K Associates Important Notes and Enclosure: 1–Valuer’s Remarks are integral part of this report and Value is assessment is subject to both of these sections. Reader of the report is advised to read all the points mentioned in these sections carefully.

| Number of Pages in the Report | Without Enclosures | 72 |
| --- | --- | --- |
| Engineering Team worked on the report | SURVEYED BY: Mr. Hemant Ambaselkar |  |
|  | PREPARED BY: PE Team |  |
|  | REVIEWED BY: HOD Valuations |  |

| LIST OF ABBREVIATIONS |  |
| --- | --- |
| VIPL | Vidarbha Industries Power Limited |
| FAR | Fixed Asset Register |
| EPC | Engineering, Procurement & Construction |
| COR | Cost-Overrun |
| COD | Commercial Operation Date |
| PPA | Power-Purchase Agreement |
| FSA | Fuel Supply Agreement |
| CERC | Central Electricity Regulatory Commission |
| GCV | Gross Calorific Value |
| RCC | Reinforced Cement Concrete |
| ESP | Electro-Static Precipitator |
| HVAC | Heating, Ventilation & Air-conditioning |
| CII | Cost Inflation Index |
| PLF | Plant Load Factor |
| GCV | Gross Calorific Value |
| STG | Steam-Turbine-Generator |
| BTG | Boiler, Turbine & Generator |
| BOP | Balance of Plant |
| CWIP | Capital Work In Progress |
| DCS | Distributed Control System |
| SPV | Special Purpose Vehicle |
| SG | Steam Generator |
| STG | Steam Turbine Generator |
| BFP | Boiler Feed Pump |
| HP | High Pressure |
| LP | Low Pressure |
| TMCR | Turbine Maximum Continuous Rating |
| BMS | Burner Management System |
| SBC | Soot Blower Control System |
| TSC | Turbine Stress Control |
| ATT | Automatic Turbine Testing |
| ATRS | Automatic Turbine Run-up System |
| EHG | Electro Hydraulic Governor Control |
| TPS | Turbine Protection System |
| WPI | Wholesale Price Index |
| PPI | Producer Price Index |

| TABLE OF CONTENTS |  |  |
| --- | --- | --- |
| SECTIONS | PARTICULARS | PAGE NO. |
| Part A | INTRODUCTION |  |
|  | - Name of the Project |  |
|  | - Purpose of the Report |  |
|  | - Brief Description of the Project |  |
|  | - Scope of the Report |  |
|  | - Documents/Data Referred |  |
| Part B | CHARACTERISTICS DESCRIPTION OF THE ASSET |  |
| Part C | AREA DESCRIPTION OF THE ASSET |  |
|  | - Total Project Land Area |  |
|  | - Building & Structures Area |  |
| Part D | INDUSTRY STATUTORY APPROVAL & NOCS DETAILS |  |
| Part E | ASSET VALUATION & COSTING ASSESSMENT |  |
|  | - Total Project Cost |  |
|  | - Land Valuation Assessment |  |
|  | - Building & Structure Valuation Assessment |  |
| Part F | PLANT & MACHINERY VALUATION ASSESSMENT |  |
|  | - Brief Description of The Plant - Power Generation Process - Fuel Supply Agreements - Power Purchase Agreement - Technology Collaborations - Information Input For Assessment - Survey Details - Power Sector in India at a Glance - Technology Overview - Valuation Procedure - Plant Strength & Advantages - Consolidated Plant & Machinery Valuation |  |
| Part G | CONSOLIDATED VALUATION ASSESSMENT OF THE PROJECT |  |
| Part H | ASSUMPTIONS/REMARKS |  |
| Part I | R.K. ASSOCIATES IMPORTANT NOTES |  |
|  | ENCLOSURES |  |
|  | 1- Valuer’s Remarks |  |
|  | 2- Rajasthan Government Circle Rates |  |
|  | 3- Valuation of Building & Civil Work |  |
|  | 4- Valuation of Plant & Machinery |  |

| PART A | INTRODUCTION |
| --- | --- |

- NAME OF THE PROJECT: Detailed Fixed Asset Valuation of 600 MW (2 X 300) MW Pulverized Coal Fired Sub Critical Thermal Power Plant set by Vidarbha Industries Power Limited (VIPL) at Butibori, MIDC Industrial Area, Nagpur District of Maharashtra, India.

600 MW (2X300 MW) Vidarbha Industries Power Limited, Butibori, MIDC Industrial Area, Nagpur District, Maharashtra.

- PURPOSE OF THE REPORT: R.K Associates has been appointed by Axis Bank Limited, Strategic Solutions Group, Worli, Mumbai for carrying out the Fixed Assets Valuation of the subject project on as-is-where-is basis.

- BRIEF DESCRIPTION OF THE PROJECT: Vidarbha Industries Power Limited has implemented coal fired subcritical thermal power projects in Butibori, MIDC Industrial Area, Nagpur District of Maharashtra, India.

This thermal power project primarily comprises of 2 units of 300MW each coal based boiler, steam turbines and generators based on sub critical technology and Balance of Plant including switch yard, Coal handling plant, Water Intake System, Ash handling plant, ECBs, Fuel Oil system, Chimney, Cooling Water system, Water treatment plant and other associated auxiliaries required for continuous, safe and reliable operation of the units.

Details pertaining to Land & Building (Building & Civil Work) and Plant & Machinery (Plant, Machinery & other fixed assets) are enumerated in different section of this report.

This Sub-critical thermal Power Plant comprising of Unit-1 & 2 having power generation capacity of 300 MW each unit. Unit wise commissioning details of the Project is mentioned below-

Table: 1

| SR. NO. | UNIT NO. | COD ACHIEVED DATE |
| --- | --- | --- |
| 1 | Unit No. 01 | 04-April-2013 |
| 2 | Unit No. 02 | 28-March-2014 |

3.1 Project Location

The Project site is located at Amgaon, Gangapur, Mandwa & Wateghat Village of Butibori, MIDC Industrial Area of Nagpur district, Maharashtra and is at a distance of around 35 km from Nagpur. The nearest major railway station is Nagpur, at a distance of 35 km from the Project site. The nearest airport is Nagpur Airport which is about 30 km from the Project site and a dedicated Railway Line has been drawn by the company from the nearest railway station i.e ~8 Km from main plant for the transportation of coal. The project can be easily access through Industrial road which is connecting NH- 44 (Sri Nagar- Kanyakumari Highway).

State

Maharashtra is a state in the western and central peninsular region of India occupying a substantial portion of the Deccan Plateau. Maharashtra is the second-most populous state in India. Spread over 307,713 sq.km (118,809 sq.mtr). It is the third-largest state by area in India. Maharashtra is bordered by the Arabian Sea to the west, the Indian states of Karnataka and Goa to the south, Telangana to the southeast and Chhattisgarh to the east, Gujarat and Madhya Pradesh to the north and the Indian union territory of Dadra and Nagar Haveli and Daman and Diu to the northwest.

District

Nagpur is the third largest city and the winter capital of the Indian state of Maharashtra. It is the 13th largest city in India by population. It is a major commercial and political center of the Vidarbha region of Maharashtra.

3.2 Land Details

For the purpose of setting up the Project, VIPL has acquired a total of 590.30 acres (23,88,856.52 sq.mtr.) of the land for the Project. Major parcel of the land parcel has been allotted by Maharashtra Industrial Development Corporation Limited (MIDC) admeasuring around 347.30 acres by virtue of lease deeds. This total land includes the Power Plant land, Land acquired for Water Corridor and for the other associated facilities.

Company has also purchased around 243 acres of land parcels from the local villagers for the development of Railway Siding for the coal transportation.

The land area taken in the Valuation assessment is purely on the basis of the information provided to us by the VIPL. We have not measured the land area on site / cizra map since the land is located in various locations & practically it is not possible to measure each and every land parcel so we have relied upon the information provided to us by the company in good faith.

3.4 Major Contract Details

This project has implemented through turnkey contracts. VIPL has appointed different EPC contractor for both the phases and for non EPC works such as green belt development, transmission line and miscellaneous works such as colony, guest house and weigh bridge were executed by the Company through separate contracts.

- Shanghai Electric Power Generation Engineering Co. (SEPGE), China - BTG equipment supplier.

- Reliance Infrastructure Limited (Reliance Infra) and Reliance Infra Projects (UK) Limited (RIPUL). - EPC contractor for complete Balance of Plant (BOP).

Other major contract details are tabulated below:-

Table: 2

| APRL PROJECT |  |
| --- | --- |
| MAJOR CONTRACT | SUPPLIER |
| BTG (Boiler Turbine Generator) | Ms. Shanghai Turbine Co. Ltd. |
| Condenser On- Load Tube Cleaning System | Ms. Qingdao Huatai Electric Equipment Co. Ltd |
| Chemical Dosing System | Ms. Milton Roy |
| Generator Transformer | Ms. Areva T&D India Ltd. |
| Reducer | Ms. Sumitomo (SHI) Cyclo Drive China, LTD |
| Control and Instrumentation | Ms. Honey Well |
| Ash Handling System | Ms. Allen Bradley |
| Vibration Monitoring System (BOP) | Ms. Rockwell Automation |
| Air Condition System Equipment | Ms. Blue Star Limited |
| Cooling Tower | Ms. Flow Tech Air Private Ltd. |

Source: Information provided by the company

The scope of work included design, engineering, manufacture, assembly, testing, packing and forwarding, supply, transportation, arrangement of comprehensive insurance, erection, testing, pre-commissioning and commissioning, conducting performance and guarantee test of Boiler, Turbine and Generator (BTG) along with its auxiliaries, station control and instrumentation, auxiliary boiler and commissioning spares for the two sub-critical units of this Plant.

3.5 Civil Work

Plant is distributed into different blocks comprising of different buildings as per their utility. These mainly comprise of Industrial Structures consisting of massive steel structural members embedded in RCC base and covered by Industrial heavy duty corrugated steel sheets. Also, some buildings are made out of brickwork and RCC with RCC Roofs. The total built-up area of the Buildings measures approximately 1,76,135 sq.mtr as per the building sheet provided by the company.

Table: 3

| APRL PROJECT |  |
| --- | --- |
| Total Area of Buildings (in Sq. mtr.) | Total Area of Buildings (in Sq. ft.) |
| 1,76,135 | 18,95,901.36 |

Source: Information provided by the company

Main sections of the Plant include Boiler House, ECB Building, ECB Control Room, Turbine Building, Coal bunker, Switchyard Control room, Control Room, Cable Vault, Fly Ash Silos, Chimney among other buildings & sections.

3.6 Water Requirement

As per the Water Balance Diagram, the consumptive water requirement with closed cooling system is about 2472.31 m³/hr. A quantity of 1,062.50m³/hr has been allocated by MIDC for Unit-1. It is being received in compartments 1A and 2A of raw water reservoir from the identified point of treated water pipeline of MIDC.

For Unit- 2, the Water Supply Agreement with the Irrigation Department, Nagpur was executed on 15-Mar-10. 1409.81 m³/hr quantity of water is received from Wadagaon Dam located at a distance of about 20 km from project site for Unit-2. VIPL has installed and commissioned a jack well and intake pumps (3x 50%) along with GRP pipeline of around 17.5 km for drawl of water from the reservoir.

3.7 Power Purchase Agreement (PPA)

VIPL has entered into a Long term Power Purchase Agreement (PPA) with R-infra to supply 100% of capacity for a period of 25-years in Mumbai and Mira Bhayandar area w.e.f. April 1, 2014.

PPA for 100% capacity with Adani Electricity Mumbai Ltd. has been terminated by AEML in April 2019, citing below-threshold availability in certain years.

While the company had challenged the validity and legality of the termination letter, it has received unfavourable rulings from MERC, and on further appeal, from APTEL. Currently, the matter is pending at the Supreme Court. Due to no clarity on availability of valid long term PPA, the Company has been unable to operate the plant which has led to deterioration in financial position and difficulty in meeting debt service obligations besides adversely impacting the health of the plant and equipment.

3.7 Fuel Supply Agreement (FSA)

The Company had a LOA for 1.23 MTPA for Unit 1. However, FSA for the same was not signed by Coal India subsidiary. The Company filed a Writ Petition, in 2017, seeking direction to CIL to execute FSA for Unit 1 of VIPL as it had a valid LOA with all milestones achieved. The matter is currently pending before Hon’ble Delhi High Court.

Meanwhile for Unit 1, the Company had received a Letter of Intent (LoI) for long-term supply of coal from WCL, under an auction conducted pursuant to SHAKTI Policy Para B(ii), for 0.8756 MTPA. Despite the termination of PPA being contested by VIPL, the LOI was cancelled by WCL citing termination of PPA by AEML, hence there is no FSA for Unit 1.

While the Company has FSA with WCL for Unit 2 for 1.11 MTPA, the same is non-operational on account of non-availability of long term/ medium term PPA. Since there is no off-take from Jan 2019, WCL has given Notice of default under FSA and claimed penalty for 2019 to till today.

Power Evacuation

Power generated from Unit-1 would be evacuated at 220 kV level into MSETCL’s network through its new Butibori-III Substation. Power generated from Unit-2 would be evacuated at 220 kV level into MSETCL’s network through the existing Butibori-I Substation located at about 2.5 Km from site. Transmission Line connecting VIPL Switchyard to MSETCL’s Butibori-III substation and Butibori-I is already commissioned.

Operations and Maintenance Contracts

As per the information provided by the company Operation & Maintenance of the project has been taken care by company itself since the plant was not in operation since 2-years. Therefore, company has not appointed any O & M Contractor.

Status of Plant during Site Survey

Our engineering team has visited the project site on December 15, 2021. During the site visit, both the units was put on rest due to protracted delays in issuance of regulatory orders and lack of fuel supply for one of the units and due to limitation due to

Our Engineering team has made the following observation during the site of the plant:

- The Condition of the plant is maintained by the company and put on rest since Jan-2019.

- During the our site survey, we came to know that the Power plant was supplying power to Reliance Infra Mumbai Power Distribution business through wheeling now this business was acquired by Adani Power and due to certain unpaid dues during M&A, Adani Power discontinued power purchase from Butibori plant.

- As per visual observation the plant, the whole plant is well maintained by the company and turbine and generators are manually operated by the company.

- The company has also faced working capital issues for the operation of the plant.

- TYPE OF REPORT: Fixed Assets Valuation Report of Project.

- PURPOSE OF THE REPORT: To Estimate & Determine the current Fair Market Value, Realizable Value, Distress Value and Liquidation Value of the Fixed Assets of the Power Plant for facilitating lenders to take further course of action on the stressed/ NPA account.

- SCOPE OF THE REPORT: To assess and determine Fair Market Value, Realizable Value, Distress Value and Liquidation Value of the Assets of 2 x 300 MW VIPL Sub-Critical Thermal Power Plant covering following points:

- Fixed Assets Valuation:

- Valuation of Land and building.

- Valuation of the Plant & Machinery of both the Units.

- Valuation of other fixed assets of the Project.

- This is just the fixed asset valuation of the project based on the cost & market approach methodologies considering the utility of the asset for the business & the company as on-ongoing concern basis. This Valuation shall not be construed as the transactional value of the Project which may be determined through Enterprise/ Business Valuation based on Income approach methodologies.

- This report only contains general assessment & opinion on the Depreciated market value  of the assets of the project found on as-is-where basis on site for which the Bank/ customer has shown & asked us to conduct the Valuation for which photographs is also attached with the report. No legal aspects in terms of ownership or any other legal aspect is taken into consideration. It doesn’t contain any due-diligence other than the valuation assessment of the property shown to us on site. Information/ data/ documents given to us by Bank/ client has been relied upon in good faith. This report doesn’t contain any other recommendations of any sort.

- DOCUMENTS/DATA REFFERED:

- Land Consolidated Sheet provided by the Company

- Copy of Land Title deeds.

- Layout Plan of the Plant

- Building Sheet having Building Details.

- Fixed Asset Register for both the units.

- Copy of Information Memorandum

- Copy of Power Purchase Agreement (PPA).

- Copy of Water Agreement (WA).

- Copy of COD Certificate for each units.

- Copies of Approvals and NOC’s from various Government agencies and departments.

- Power Plants references available in public domain

| PART-B | VALUATION ASSESSMENT AS PER SBI FORMAT |
| --- | --- |

| Name & Address of Branch: | Axis Bank Limited, Strategic Solutions Group, Worli, Mumbai. |
| --- | --- |
| Name of Customer (s)/ Borrower Unit | M/s. Vidarbha Industries Power Limited |

| I. | GENERAL |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Purpose for which the valuation is made |  |  | For Distress Sale of mortgaged assets under NPA a/c |  |  |  |
|  |  | Date of inspection |  | 14 April 2021 |  |  |  |
|  |  | Date on which the valuation is made |  | 14 October 2021 |  |  |  |
|  | List of documents produced for perusal |  |  | Documents Requested | Documents Provided |  | Documents Reference No. |
|  |  |  |  | Total 04 documents requested. | Total 04 documents provided. |  | 04 |
|  |  |  |  | Allottment Papers | Project Approval Documents |  | --- |
|  |  |  |  | Property Title document | Lease Deed |  | --- |
|  |  |  |  | Approved Map | Fixed Asset Register |  | --- |
|  |  |  |  | Project Approval Documents | Different Approval Documents Available |  | --- |
|  | Name of the owner/s |  |  | M/s. Vidarbha Industries Power Limited |  |  |  |
|  | Address and Phone no. of the owner/s |  |  | Regd. Office: H Block, 1st Floor, Dhirubhai Ambani Knowledge City, Navi Mumbai – Maharashtra |  |  |  |
|  | Brief description of the property |  |  | M/s. Vidarbha Industries Power Limited has set up a 600 MW (2 X 300) pulverized coal fired Sub-Critical Thermal Power Plant at Butibori Industrial Area, Nagpur District of Maharashtra, India. This is a Sub-Critical pulverized coal fired Power Plant. The Plant comprises of 2 Units of 300 MW each. Unit #1 has been commissioned & it successfully achieved COD on April 4th, 2013 and Unit #2 has achieved its COD on March 28th, 2014. Other Project details are already described in ‘Brief description of the Project’ under Part-A Introduction section. The Project site is located at Amgaon, Gangapur, Mandwa & Wateghat Village of Butibori, MIDC Industrial Area of Nagpur district, Maharashtra and is at a distance of around 35 km from Nagpur. The nearest major railway station is Nagpur, at a distance of 35 km from the Project site. The nearest airport is Nagpur Airport which is about 30 km from the Project site and a dedicated Railway Line has been drawn by the company from the nearest railway station i.e ~8 Km from main plant for the transportation of coal. The project can be easily access through Industrial road which is connecting NH- 44 (Sri Nagar- Kanyakumari Highway) |  |  |  |
|  | Location of property |  |  |  |  |  |  |
|  |  | Plot No. / Survey No. |  | Different as per different Deeds |  |  |  |
|  |  | Door No. |  | NA |  |  |  |
|  |  | T. S. No. / Village |  | Village: Amgaon, Gangapur, Mandwa & Wateghat |  |  |  |
|  |  | Ward / Taluka |  | Hingra |  |  |  |
|  |  | Mandal / District |  | Nagpur |  |  |  |
|  |  | Date of issue and validity of layout of approved map / plan |  | Architect Approved Map – 12/05/2008 |  |  |  |
|  |  | Approved map / plan issuing authority |  | Architect Approved Map |  |  |  |
|  |  | Whether genuineness or authenticity of approved map / plan is verified |  | Genuineness of the approved map is not authenticated by us. But the document provided have a stamp from the authorized architect. |  |  |  |
|  |  | Any other comments by our empanelled Valuers on authenticity of approved plan |  | No |  |  |  |
|  | Postal address of the property |  |  | Plot No. D-3, Butibori, MIDC Industrial Area, Nagpur, Maharashtra |  |  |  |
|  |  | City / Town |  | Village :- Amgaon, Gangapur, Mandwa & Wateghat |  |  |  |
|  |  | Residential Area |  | No |  |  |  |
|  |  | Commercial Area |  | No |  |  |  |
|  |  | Industrial Area |  | Notified Industrial Area |  |  |  |
|  | Classification of the area |  |  |  |  |  |  |
|  |  | High / Middle / Poor |  | Middle |  |  |  |
|  |  | Urban / Semi Urban / Rural |  | Rural |  |  |  |
|  | Coming under Corporation limit/ Village Panchayat / Municipality |  |  | MIDC – Maharashtra Industrial Development Corporation |  |  |  |
|  | Whether covered under any State / Central Govt. enactments (e.g. Urban and Ceiling Act) or notified under agency area / scheduled area / cantonment area |  |  | Not known to us |  |  |  |
|  | In case it is an agricultural land, any conversion to house site plots is contemplated |  |  | The subject land is leased from MIDC for setting up power plant unit. |  |  |  |
|  | Boundaries of the property |  |  |  |  |  |  |
|  | Are Boundaries matched |  |  | Since, The land is spread over a very large area so the boundaries can not be identified easily |  |  |  |
|  | Directions |  | As per Lease Deed |  |  | Actual found at Site |  |
|  | North |  | ---- |  |  | Maharashtra Enviro Power Ltd. (MEPL)/ Plot No. CHW-01 |  |
|  | South |  | ---- |  |  | M/s. Fabtech Plot No. D-71 MIDC Butibori |  |
|  | East |  | ---- |  |  | Bharat Petroleum LPG Plant, Plot No. D-1 |  |
|  | West |  | ---- |  |  | M/s Gammco, Plot No. J-9 |  |
| 14.1 | Dimensions of the site |  | A |  |  | B |  |
|  |  |  | As per the Deed |  |  | Actuals |  |
|  | North |  | Not mentioned in the documents |  |  | Not measurable at the site |  |
|  | South |  | Not mentioned in the documents |  |  | Not measurable at the site |  |
|  | East |  | Not mentioned in the documents |  |  | Not measurable at the site |  |
|  | West |  | Not mentioned in the documents |  |  | Not measurable at the site |  |
| 14.2 | Latitude, Longitude & Co-ordinates of Industrial Property |  |  | 20°55'31.2"N 78°56'00.6"E |  |  |  |
| 15. | Extent of the site |  |  | Total Project Land Area: 23,88,856.52 m2 (590.32 acres) |  |  |  |
| 16. | Extent of the site considered for valuation (least of 14 A & 14 B) |  |  | Total Project Land Area: 23,88,856.52 m2 (590.32 acres) |  |  |  |
| 17. | Whether occupied by the owner / tenant? |  |  | Owner Company |  |  |  |
|  | If occupied by tenant, since how long? |  |  | Not applicable |  |  |  |
|  | Rent received per month. |  |  | Not applicable |  |  |  |

| II. | CHARACTERISTICS OF THE SITE |  |
| --- | --- | --- |
|  | Classification of locality | Notified Industrial Area |
|  | Development of surrounding areas | Notified Industrial area so all adjacent land use is Industrial |
|  | Possibility of frequent flooding / sub-merging | No |
|  | Feasibility to the Civic amenities like school, hospital, bus stop, market etc. | Civic amenities such as School is around 05km. and amenities such as Hospital and Market are at a distance of around 15km. from the subject property. The nearest railway station is at a distance of around 35km. |
|  | Number of Floors | Industrial structures |
|  | Type of Structure | RCC & GI Shed |
|  | Type of use to which it can be put | Power Plant Unit |
|  | Any usage restriction | The subject land is allotted for setting up of Power Plant Unit by MIDC. |
|  | Is plot in town planning approved layout? | No |
|  | Corner plot or intermittent plot? | Irrelevant for the subject plot as this is a huge land parcels. |
|  | Road facilities | Yes |
|  | Type of road available at present | Bitumen surface road |
|  | Width of road – is it below 20 ft. or more than 20 ft. | Above 20.mtr. |
|  | Is it a land – locked land? | No |
|  | Water potentiality | Yes, from ground water and pipeline connectivity to river. |
|  | Underground sewerage system | Yes |
|  | Is power supply available at the site? | Yes |
|  | Advantage of the site | The subject project is in developed industrial area of Nagpur. |
|  | Special remarks, if any, like threat of acquisition of land for public service purposes, road widening or applicability of CRZ provisions etc. (Distance from sea-coast / tidal level must be incorporated) | None, in our view |

| PART- C | VALUATION OF LAND |
| --- | --- |

|  | Size of Plot | 23,88,856.52 m2 (590.32 acres) |
| --- | --- | --- |
|  | North & South | No information available in the provided documents. |
|  | East & West | No information available in the provided documents. |
|  | Total extent of the plot | 23,88,856.52 m2 (590.32 acres) |
|  | Area adopted on the basis of | Property documents only since site measurement couldn't be carried out as the subject plot is of ultra large size and irregular in shape. |
|  | Remarks & observations, if any | Area adopted on the basis Copy of deeds and consolidated area sheet provided to us by the company. |
|  | Prevailing market rate (Along with details /reference of at least two latest deals/transactions with respect to adjacent properties in the areas) | Refer to the land assessment section below: - |
|  | Guideline rate (an evidence thereof to be enclosed) | Refer to the sheet attached below |
|  | Assessed / adopted rate of valuation | Refer to the sheet attached below |
|  | Estimated value of land (A) | Refer to the sheet attached below |

LAND VALUATION ASSESSMENT

METHODOLOGY ADOPTED: For the purpose of setting up the Project, VIPL has acquired a total of 590.30 acres (23,88,856.52 sq.mtr.) of the land for the Project. Major of the land parcel i.e. 347.30 acres of land parcels has been allotted from Maharashtra Industrial Development Corporation Limited (MIDC) by the virtue of Lease Deeds for the development of Power project. This total land includes the Power Plant land, Land acquired for Water Corridor and for the other associated facilities.

Company has also purchased around 243 acres of land parcels from the local villagers for the development of Railway Siding for the coal transportation.

The detailed break-up of the total land acquired/purchased by VIPL is given below:

Table: 4

Land Valuation assessment is done considering the Land use for Power Project Land only as its highest & best use since the transaction of this land will be done based on the established Project only and separation of it from the Project will be virtually impossible, at least up to the complete economic life cycle of this Plant which is taken as 25-years and also due to the location no other more productive use can be thought of.

Overall Valuation assessment is done based on the fact that if any new promoter/ buyer would plan to setup a similar Plant today at same location and same size then what will the similar size of land acquisition will cost to him as on date of valuation. To acquire ~590.30 acres (23,88,856.52 sq.mtr.) of land parcel, one would either go to Govt. to provide the land for Industry or would approach individual land owners to purchase their land portions.

The land we have considered for the Valuation assessment is ~590.30 acres (23,88,856.52 sq.mtr.) of land area as per the information/ data supplied by the company, which is mortgaged to the lenders.

Fragmentation sale of a large land may have different values. While assessing the Valuation of the land in this Valuation Report, it is considered as on-is-where basis for the purpose it is used for which was found at the time of site survey.

Circle Rate Value: Circle rate of the land is calculated based on Department of Registration & Stamps Government of Maharashtra In the procedure of assessment following points are taken into consideration:

- This Project Land is used for Industrial purpose. The major land of the project is lying in the villages Amgaon, Gangapur, Mandwa & Wateghat. The circle rates of land located at Village- at Amgaon, Gangapur, Mandwa & Wateghat is Rs.6,600/- per sq.mtr. A Copy of the guideline rate is annexed with the report for reference.

- The subject plant is in Butibori Industrial area of Maharashtra Industrial Development Corporation (MIDC) where the present allotment rate is Rs.1,595/- per sq.mtr. A Copy of the guideline rate is annexed with the report for reference.

Fair Market Rate Value: Market Value of this Project land would be the value which any new promoter company will be spending the amount in procuring the equal measurement of the land parcel if it wants to setup a similar plant today on the date of the valuation.

Hence Market value of the land is assessed based on the significant local enquiries, reference of historical sales comparable searched by our team and making rational factors of adjustment on it based on current market situation & condition. In the procedure of assessment following points are taken into consideration:

- The company has signed a MoU with Maharashtra government for the development of power project at the aforesaid address. Therefore, the majority of land parcel of the plant has been allotted by Maharashtra Industrial Development Corporation (MIDC) for the development of power project.

- So, any new promoter Company will have to purchase/acquire this type of land from MIDC only if it wants to setup a similar plant as on date.

- Historical land transaction information for this land is also referred. VIPL has capitalized the Land value in the FAR which includes site development charges also as Rs.236.46 Cr. which is translated as Rs.40,05,727/- per acres.

- As per the Notification of Butibori Industrial area, Maharashtra Industrial Development Corporation (MIDC), the allotment rate for the industrial land is Rs.1,595/- per sq.mtr. As per the information available on public domain of MIDC, no such large land parcel is available as of now.

- Significant recent sales comparable & market rate enquiries were made from local villagers, village tehsildar & land property dealers of the village where major land is acquired.

- Local Micro Market Survey: According to him, the asking price of land is averagely ranging from Rs.3,000/- to Rs.4,000/- per sq.mtr. for the small size of land.

- During the site visit our engineering team has conducted a market survey of the area near to the plant and its suburbs. After interaction with local habitants, the rates which came into our knowledge ranges between Rs.3,000/- to Rs.4,000/- per sq.mtr for Land nearby Power Plant depending upon the village and the location of the land.

- This is a developing industrial area and there is already many industries setup with in the vicinity of plant due to this demand of the land is increased for the small & mid-size of the land.

- The subject land parcel is huge and allotted by the MIDC for the development of Power Projects.

- Therefore, considering all the factors like size, demand, present economic condition of power industry, and the location we have considered the allotment rate of Rs.1,595/- per sq.mtr as market rate for the subject land, since the power plant is situated in the notified industrial area of MIDC and the subject land can only be allotted through MIDC as per the aforesaid allotment rate of the subject industrial area, and a copy of the same has been attached in the annexure below.

- In addition to this basic rates of the land, around 10% premium is added on this rate which covers the cost & effort consideration to cover administrative cost, effort towards land consolidation & land conversation charges.

- The Valuation of the 30-year leased from SICOM for railway siding is considering to the parity of other leased land allotted by MIDC and rates for the free hold, we have taken same rate as lease hold land.

We have also weighed the land rate on the basis of land acquisition act-2013 and award value comes on very higher side in comparison to the prevailing market rate considering premium also. Since this project land is already an acquired land and formed a large land parcel and in the present scenario when Power sector is facing demand & financial crunch, so definitely any buyer would go for negotiation on prevailing market rate based on all of the above prevailing factors. Therefore, being on the conservative side we have taken land value on the basis of ‘Market Comparable Sales Approach’ and further calculated based on the lease hold rights for the balance period of 18 years.

- LAND VALUATION CALCULATION FOR LEASE HOLD LAND:

Table: 5

| Fair Market Value of Project Land | Rs.419.12 Cr. |
| --- | --- |

| PART- D | VALUATION OF BUILDING |
| --- | --- |

|  | Technical details of the building |  | Construction done using professional contractor workmanship based on architect plan. |
| --- | --- | --- | --- |
|  |  | Type of Building (Residential / Commercial/ Industrial) | Industrial |
|  |  | Type of construction (Load bearing / RCC/ Steel Framed) | RCC & GI Shed |
|  |  | Year of construction | Around 2010-2012 |
|  |  | Number of floors and height of each floor including basement, if any | Refer to the sheet attached below |
|  |  | Plinth area floor-wise | Refer to the sheet attached below |
|  |  | Condition of the building | Good |
|  |  | Interior Finishing | POP punning, Simple Plastered Walls, Designer false ceilingin office areas |
|  |  | Exterior Finishing | Neatly plastered & putty coated walls, Glass facade, Architecturally designed & elevated |
|  | Status of Building Plans/ Maps |  | Architect Approved map |
|  |  | Date of issue and validity of layout of approved map / plan | Architect Approved Map – 12/05/2008 |
|  |  | Is Building as per approved Map | Cannot comment since no area statement chart mentioned in the plant layout plan |
|  |  | Whether genuineness or authenticity of approved map / plan is verified | No |
|  |  | Any other comments by our empaneled valuers on authentic of approved plan | No |
|  |  | Details of alterations/ deviations/ illegal construction/ encroachment noticed in the structure from the original approved plan | None |

Table: 6

|  | SPECIFICATIONS OF CONSTRUCTION (FLOOR-WISE) IN RESPECT OF |  |  |
| --- | --- | --- | --- |
| S.No. | Description | Ground floor | Other floors |
|  | Foundation | RCC | NA |
|  | Ground Floor | Please refer to sheet attached above. | Please refer to sheet attached above. |
|  | Superstructure | Please refer to sheet attached above. | Please refer to sheet attached above. |
|  | Joinery / Doors & Windows (please furnish details about size of frames, shutters, glazing, fitting etc. and specify the species of timber) | Aluminum flushed doors & windows & Wooden frame & panel doors | Aluminum flushed doors & windows & Wooden frame & panel doors |
|  | RCC works | Completed | Completed |
|  | Plastering | Completed | Completed |
|  | Flooring, Skirting, dadoing | Completed | Completed |
|  | Special finish as marble, granite, wooden paneling, grills, etc. | No | No |
|  | Roofing including weather proof course | Completed | Completed |
|  | Drainage | Yes | NA |

| S.No. | Description |  | Ground floor | Other floors |
| --- | --- | --- | --- | --- |
|  | Compound wall |  | Yes | NA |
|  | Height |  | Approx. 10 ft. | NA |
|  | Length |  | -- | NA |
|  | Type of construction |  | RCC | NA |
|  | Electrical installation |  |  |  |
|  | Type of wiring |  | Internal & External | Internal & External |
|  | Class of fittings (superior / ordinary / poor) |  | Superior | Superior |
|  | Number of light points |  | NA | NA |
|  | Fan points |  | NA | NA |
|  | Spare plug points |  | NA | NA |
|  | Any other item |  | NA | NA |
|  | Plumbing installation |  |  |  |
|  |  | No. of water closets and their type | NA | NA |
|  |  | No. of wash basins | NA | NA |
|  |  | No. of urinals | NA | NA |
|  |  | No. of bath tubs | NA | NA |
|  |  | Water meter, taps, etc. | NA | NA |
|  |  | Any other fixtures | NA | NA |

| III | EXTRA ITEMS |
| --- | --- |

|  | Portico | NA |
| --- | --- | --- |
|  | Ornamental front door | NA |
|  | Sit out/ Verandah with steel grills | Already considered in building sheet |
|  | Overhead water tank | NA |
|  | Extra steel/ collapsible gates | Already considered in building sheet |
|  | Total (C) | NA |

| IV | AMENITIES |
| --- | --- |

|  | Wardrobes | Already considered in building sheet |
| --- | --- | --- |
|  | Glazed tiles | Already considered in building sheet |
|  | Extra sinks and bath tub | Already considered in building sheet |
|  | Marble / Ceramic tiles flooring | Already considered in building sheet |
|  | Interior decorations | Already considered in building sheet |
|  | Architectural elevation works | Already considered in building sheet |
|  | Paneling works | NA |
|  | Aluminum works | NA |
|  | Aluminum hand rails | Already considered in building sheet |
|  | False ceiling | Already considered in building sheet |
|  | Total (D) | NA |

| V | MISCELLANEOUS |
| --- | --- |

|  | Separate toilet room | Already considered in building sheet |
| --- | --- | --- |
|  | Separate lumber room | NA |
|  | Separate water tank/ sump | NA |
|  | Trees, gardening | Already considered in building sheet |
|  | Total (E) | NA |

| VI | SERVICES |
| --- | --- |

|  | Water supply arrangements | Already considered in building sheet |
| --- | --- | --- |
|  | Drainage arrangements | Already considered in building sheet |
|  | Compound wall | Already considered in building sheet |
|  | C. B. deposits, fittings etc. | Already considered in building sheet |
|  | Pavement | Already considered in building sheet |
|  | Total (F) | NA |

VALUATION OF CIVIL BUILDINGS & STRUCTURAL WORKS

- BUILDING & STRUCTURES VALUATION ASSESSMENT:

- METHODOLOGY ADOPTED: The fair market value of the building on the date of valuation is its cost of reproduction on that date less the depreciation & other deterioration deductions from the date of completion of the building to the date of its valuation.

- We have only considered the Buildings for the Valuation which is under the Buildings head in the Fixed Asset Register provided to us to avoid duplicity of value.

- Building/ Civil Structures related to the Plant & Machinery are not shown separately under building and is not considered for the Valuation since these are the part of Plant & Machinery and is capitalized in the Plant & Machinery head in the Fixed Asset Register provided to us by the company.

- In normal exercise, the benchmark cost for the Building/ civil structure as per CERC is around Rs.0.8 Cr. to 1.20 Cr. per MW as per index. But in provided FAR, the company has shown only 0.62 Cr. per MW and according to the company, they have considered most of the structure in Plant & machinery section. Hence the Valuation of the civil structure/ building has been carried out on the basis of Construction Cost Indices (CCI) and not on the basis of plinth area to avoid the duplicity of the value of the civil work.

- Reproduction Cost of the Building/ Civil Structures is calculated based on the Construction Cost Indices (CCI) provided by the Construction Industry Development Council of Planning Commission, Govt. of India. Copy of the Index is annexed with the report for reference.

- Reproduction Cost of the structure is calculated from the date of capitalization to the date of valuation.

- Depreciation is charged on the Buildings & structures considering the life of civil structure/ building as 10 to 40 years as per the Chart of Companies Act-2013.

- Our engineering team did the sample measurement of some of the building structures which was found more or less to be same.

- The condition of the buildings and structures found to be average during the site visit.

Table: 7

| Fair Market Value of Building & Other Civil Structures | Rs.224.03 Cr. |
| --- | --- |

| PART D | INDUSTRY STATUTORY APPROVAL & NOC’S DETAILS |
| --- | --- |

Table: 8

| S.No. | REQUIRED APPROVALS | REFERENCE NO./ DATE | STATUS (Approved/ Applied For/ Pending) |
| --- | --- | --- | --- |
|  | Approved Layout plant approved by Industrial Safety & Healthy Department, Nagpur | Ref. No. PLAN/SDT/SAK/178-12/579-82 Dated : 15.02.2013 | Obtained |
|  | Plant layout plant approved by MIDC, Nagpur | Letter No. DB/PALN2007 Dated : 10.04.2012 | Obtained |
|  | Factory License issued from Factory Department, Maharashtra | Memo No. 1881/NGP/2(M)(I) Dated : 07.12.2012 | Obtained |
|  | NOC for Factory Building | Memo No. MIDC/FIRE/NOC/1815/of 13 Dated : 20.07.2013 | Obtained |
|  | NOC for Pollution issued from Pollution control Board, Mumbai, Maharashtra | Consent No. Format 1.0/BO/CAC-Cell / Autorenewal/CAC-0101 Dated : 06.02.2017 | Obtained |
|  | License for storage of Carbon Dioxide, Hydrogen & Nitrogen gas in Cylinder at plant issued from Ministry of Commerce & Industry, Petroleum & Explosives Safety Organization | No. G/WC/MH/06/2331(G31448) Dated: 17.07.2020 | Obtained |
|  | License for storage of Petroleum Class-C at plant issued from Ministry of Commerce & Industry, Petroleum & Explosives Safety Organization | No. P/HQ/MH/15/6068(P250859) Dated: 14.10.2017 | Obtained |

Observations: Project meets preliminary necessary compliance statutory approvals.

| PART F | PLANT & MACHINERY VALUATION ASSESSMENT |
| --- | --- |

- BRIEF DESCRIPTION OF THE PLANT: This section of the Valuation report is prepared for the Project Plant & Machinery with a capacity of 2x300 MW Pulverized Coal Based Sub-Critical Thermal Power Plant setup by Vidarbha Industries Power Limited (VIPL). Sub-critical technology is being used for the generation of power in this plan.

This is a Sub-Critical pulverized coal fired Power Plant. The Plant comprises of 2 Units of 300 MW each. Unit #1 has been commissioned & it successfully achieved COD on April 4th, 2013 and Unit #2 has achieved its COD on March 28th, 2014.

At the time of site survey, the whole plant was shut down due to protracted delays in issuance of regulatory orders and lack of fuel supply for one of the units. And also during our site survey we came to know that R-infra Power distribution business was taken over by Adani Power and during M & A some certain unpaid dues pending. Therefore, Adani discontinued power purchase from Butibori plant.

Project Company had awarded the EPC contract for the Project to a consortium of Reliance Infrastructure Limited (Rel Infra), the principal Contractor, and Reliance Infra Projects (UK) Limited (RIPUL), its off-shore affiliate. Reliance Infrastructure would be responsible for coordinating with RIPUL to perform the obligations of the contractors under the contract. The EPC contractor has already placed the BTG contract with Shanghai Electric Corporation (SEC), China, an OEM of repute.

The scope of work includes design, engineering, manufacture, assembly, testing, packing and forwarding, supply, transportation, arrangement of comprehensive insurance, erection, testing, pre-commissioning and commissioning, conducting performance and guarantee test of Boiler, Turbine and Generator (BTG) along with its auxiliaries, station control and instrumentation, auxiliary boiler and commissioning spares for the three sub-critical units of this Plant.

Project Company has involved multiple contractors for erection of Balance of Plant (BOP) and Civil works of the Project and further company has awarded the major BOP contracts and Non-EPC contractors to various contractors.

Main machinery of the plant includes Boiler, Turbine, Generator, Coal Handling Plant, Water Treatment Plant, Switchyard, Transmission line, Water pipeline system to bring raw water to the plant, and other auxiliary machinery for running the plant.

Power generated from Unit-1 would be evacuated at 220 kV level into MSETCL’s network through its new Butibori-III Substation. Power generated from Unit-2 would be evacuated at 220 kV level into MSETCL’s network through the existing Butibori-I Substation located at about 2.5 Km from site. Transmission Line connecting VIPL Switchyard to MSETCL’s Butibori-III substation and Butibori-I is already commissioned.

This Power plant is compliant with all the required preliminary necessary statutory approvals, licenses, NOCs like EC Clearance, Factory License, NOC for Petroleum & Explosive storage etc.

- POWER GENERATION PROCESS:

- First, the pulverized coal is burnt in the furnace of steam boiler.

- High pressure steam is produced in the boiler.

- This Steam is then passed through the super heater, where it is further heated up.

- This heated steam is then entered into a turbine at high speed.

- In the turbine, this steam at high pressure rotates the turbine blades i.e., potential energy of the high pressured steam is converted into mechanical energy.

- After rotating the turbine blades, the Steam loses its high pressure, thereafter the steam is   passed out of turbine blades and enters the condenser.

- In the condenser the cold water is circulated with help of pump which condenses the low pressure wet steam.

- This condensed water is then further supplied to low pressure heater which increases the temperature of the feed water, it is then again heated in a high pressure heater where the high pressure of steam is used for heating.

- The turbine in thermal power station acts as a prime mover of the alternator.

MAIN STEAM

167KG/CM2,540°C

REHEATED STEAM

32KG/CM2,540°C

300 MW,

MAKE:M/s Shanghai Turbine Works

353 MVA GENERATOR

275°C

184.5KG/CM2,

360°C

TYPICAL COAL THERMAL POWER STATION

- FUEL SUPPLY AGREEMENTS (FSA):

The Company had a LOA for 1.23 MTPA for Unit 1. However, FSA for the same was not signed by Coal India subsidiary. The Company filed a Writ Petition, in 2017, seeking direction to CIL to execute FSA for Unit 1 of VIPL as it had a valid LOA with all milestones achieved. The matter is currently pending before Hon’ble Delhi High Court.

Meanwhile for Unit 1, the Company had received a Letter of Intent (Loi) for long-term supply of coal from WCL, under an auction conducted pursuant to SHAKTI Policy Para B(ii), for 0.8756 MTPA. Despite the termination of PPA being contested by VIPL, the LOI was cancelled by WCL citing termination of PPA by AEML, hence there is no FSA for Unit 1.

While the Company has FSA with WCL for Unit 2 for 1.11 MTPA, the same is non-operational on account of non-availability of long term/ medium term PPA. Since there is no off-take from Jan 2019, WCL has given Notice of default under FSA and claimed penalty for the year 2019-20:          ~ Rs. 90 Crore. Towards recovery of the said penalty, BGs aggregating to ~Rs. 15 Cr were invoked by WCL. It has asked VIPL to restore the BGs, but the same has not been done, which constitutes a default under FSA, potentially attracting FSA termination.

- POWER PURCHASE AGREEMENTS (PPA):

VIPL has entered into a Long term Power Purchase Agreement (PPA) with R-infra to supply 100% of capacity for a period of 25-years in Mumbai and Mira Bhayandar area w.e.f. April 1, 2014.

PPA for 100% capacity with Adani Electricity Mumbai Ltd. has been terminated by AEML in April 2019, citing below-threshold availability in certain years.

While the company had challenged the validity and legality of the termination letter, it has received unfavorable rulings from MERC, and on further appeal, from APTEL. Currently, the matter is pending at the Supreme Court. Due to no clarity on availability of valid long term PPA, the Company has been unable to operate the plant which has led to deterioration in financial position and difficulty in meeting debt service obligations besides adversely impacting the health of the plant and equipment.

- TECHNOLOGY COLLABORATIONS:

This project has implemented through turnkey contracts. VIPL has appointed different EPC contractor for both the phases and for non EPC works such as green belt development, transmission line and miscellaneous works such as colony, guest house and weigh bridge were executed by the Company through separate contracts.

- Shanghai Electric Power Generation Engineering Co. (SEPGE), China - BTG equipment supplier.

- Reliance Infrastructure Limited (Reliance Infra) and Reliance Infra Projects (UK) Limited (RIPUL). - EPC contractor for complete Balance of Plant (BOP).

Table: 9

| APRL PROJECT |  |
| --- | --- |
| MAJOR CONTRACT | SUPPLIER |
| BTG (Boiler Turbine Generator) | Ms. Shanghai Turbine Co. Ltd. |
| Condenser On- Load Tube Cleaning System | Ms. Qingdao Huatai Electric Equipment Co. Ltd |
| Chemical Dosing System | Ms. Milton Roy |
| Generator Transformer | Ms. Areva T&D India Ltd. |
| Reducer | Ms. Sumitomo (SHI) Cyclo Drive China, LTD |
| Control and Instrumentation | Ms. Honey Well |
| Ash Handling System | Ms. Allen Bradley |
| Vibration Monitoring System (BOP) | Ms. Rockwell Automation |
| Air Condition System Equipment | Ms. Blue Star Limited |
| Cooling Tower | Ms. Flow Tech Air Private Ltd. |

The scope of work included design, engineering, manufacture, assembly, testing, packing and forwarding, supply, transportation, arrangement of comprehensive insurance, erection, testing, pre-commissioning and commissioning, conducting performance and guarantee test of Boiler, Turbine and Generator (BTG) along with its auxiliaries, station control and instrumentation, auxiliary boiler and commissioning spares for the two sub-critical units of this Plant.

- INFORMATION INPUT FOR ASSESSMENT: One common fixed asset register was provided to our team having capitalization date and capitalization cost for the project together as on March- 2021

Based on the category of items viz. Building, Plant & Machinery, Leasehold Land, Freehold Land, Furniture & Fittings, Office Equipment, Motor Vehicles and Infrastructure our team has separated the lists based on the category of assets for separate evaluation.

Along with these sheets, our team was provided with a fixed asset register having items such as Land, Office Equipment’s, Furniture & Fixtures etc. All this information was relied upon to be true and is made the base of the Valuation assessment.

- SURVEY DETAILS:

- This plant has been surveyed by our Engineering team. Site inspection was done in the presence of plant representatives.

- Only major machinery, equipment & sections were cross checked as per the documents provided to us and sample random checking was done of the overall plant.

- Team visited all the different blocks of the plant.

- Photographs have also been taken of all the items/machinery installed in the Plant.

- At the time of site survey, both the units was shut down due to the some issue in between Power Producer Company and power Purchaser Company.

- Machine condition reported in this report is only based on the visual observation. No mechanical/ functional testing is carried out to know the efficiency or condition of the machines. The project was operational at the time of site survey.

- This valuation is done only on the basis of the physical existence of the assets during the site survey rather than their technical expediency.

- Site survey was conducted on 15 December 2021.

Some of the photographs clicked during the site visit are attached below:

TG – UNIT 1

COAL MILL FEEDER

SILO

CENTRAL CONTROL ROOM

GENERATOR

MAGNETIC COAL SEPERATOR

SERVICE BUILDING

WAGON TRIPPLER

CHIMNEY

RAILWAY SIDINGS

OFFICE SPACE

COAL YARD

- POWER SECTOR IN INDIA AT A GLANCE:

- Utility Power & Growth of Installed capacity India

Table: 10

- Total Installed Capacity In India

Table: 11

Source: https://en.wikipedia.org/wiki/Electricity_sector_in_India

Indian power sector is undergoing a significant change that has redefined the industry outlook. Sustained economic growth continues to drive electricity demand in India. The Government of India’s focus on attaining ‘Power for all’ has accelerated capacity addition in the country. At the same time, the competitive intensity is increasing at both the market and supply sides (fuel, logistics, finances, and manpower).

Total installed capacity of power stations in India stood at 3,82,151 Megawatt (MW) as of March-2021. On the contrary, coal-based power generation capacity in India, which currently stands at 2,09,294 MW is expected to reach 330-441 GW by 2040.

- TECHNOLOGY OVERVIEW: Pulverized coal combustion (PC) is the most widely used technology in coal-fired power plants globally. The technology’s developments in the past decades have primarily involved increasing plant thermal efficiencies by raising the steam pressure and temperature. Based on the differences in temperature and pressure, the technology is categorized into three tiers subcritical, supercritical (SC) and ultra-supercritical (USC).

Table: 12

- SALES TRANSACTIONAL PROSPECTS OF SUCH PLANTS: Infrastructure assets like power plants are of great value for the nation and these have much greater socio-economic value, in addition to its core Asset value.

In case of sale transactions, such assets can be divested only through strategic sales by way of acquisition or takeovers by the large conglomerates who are either already into the same business line or any other large corporates who may have business interest to diversify for entering into such sectors if they are already not into it.

This will be a good deal for any buyer to acquire this plant since this plant is tied up with complete PPA & FSA. Therefore, it will be a good option for major domestic & foreign big shots in energy sector to take up stakes in this valuable asset. However, the evaluation of the transactional value can be done based on its Enterprise/ Business value which will largely depend on the future income projections and its unit pricing and operational cost.

- VALUATION PROCEDURE: For arriving at fair market value of P&M asset our engineering team has rationally applied the mixture of ‘cost approach (depreciated replacement cost)’.

- Main reference point for the assessment of the Valuation of this Plant is Fixed Asset register dated March-2021 provided by the company which is relied upon.

- Core P&M Asset Valuation is done keeping in mind various factors like technology used, machines availability, its condition, average age, maintenance & service and parts replacement availability of the machines and more importantly demand in the market.

- Machinery items in this plant are specific purpose machines used for the thermal power plant which limits the realizable value to the particular purpose only.

- From the fixed asset sheet two key inputs, year of manufacture & original capitalization cost are taken which plays vital role in evaluating used Plant & Machinery valuation.

- Capitalization cost includes Design, erection, procurement, installation & commissioning charges as well.

- In provided FAR, soft cost incurred during the Project commissioning like Pre-operative expenses (insurance, taxes, freight), Finance Cost, Bank interest, charges etc. is capitalized with hard cost and breakup of hard cost & soft cost was not provided to us.

- For evaluating depreciation, Central Electricity Regulatory Commission (CERC) Guidelines & Chart of Companies Act-2013 for ascertaining useful life of different types of machines are followed. Useful life of Primary machines of the Plant like Boiler, Turbine, Generator, Coal Handling System etc. is taken as 25 years. For other auxiliary machinery & equipment average life varies from 15- 25 years.

- Package wise capital cost incurred in the Project up to COD is also referred during evaluation process.

- For evaluating the current market value of machines, general CERC benchmark and other similar plants cost information is also gathered & referred to.

- Individual postings for prospective sale/ stake, acquisition of such Projects in the market are also explored to arrive at the correct depreciated replacement value of the machines.

- $ rate fluctuation is not considered separately in our assessment since the adjustment of this fluctuation in the overall cost of the Project is already capitalized by the company in FAR.

- This is a sub-critical technology based plant which is currently in high demand as per the market research.

- On the depreciated value, we have taken physical & functional wear-n-tear factor on main plant equipment based on the condition of the asset at the time of site survey.

- This valuation would be more reasonable if anyone would buy this plant in as is installed condition. Also, Valuation may differ in case these assets are sold in dismantled/Uninstalled condition. This will depend on its dismantling/uninstalling cost and also on the views of the buyer and seller.

- In the provided FAR, a list of vehicles was provided. No detailed information regarding the Manufacture Name, Type, Model, Registration Certification (RC) etc. was provided to us on request. We have done the assessment of these vehicles relying on the fact that the vehicles are in the name of Project Company. Bank is advised to take the RC and other documents related to these vehicles.

- This Valuation should be referred based on the macro analysis of the asset considering it in totality as a whole and not based on the micro, component or item wise analysis. Analysis done is a general assessment and is not investigative in nature.

- All the gathered information & data is further collated & analyzed and obsolescence/ deterioration factor is applied to arrive at the final value of the machines that is shown in the separate sheet.

- PLANT STRENGTH & ADVANTAGES:

- The subject plan is in developed industrial area of Nagpur.

- The power plant is newly setted up and achieved its COD in on April 4th, 2013 and March 28th, 2014 respectively.

- PLANT DRAWBACKS:

- The subject plan doesn’t have PPA.

- The power plant doesn’t have any FSA.

- The land allotted to power plant can only be used for power generation unit so if transaction will happen on piecemeal basis so land parcel will not fetch any value.

- CONSOLIDATED PLANT & MACHINERY VALUATION: Consolidated valuation inventory sheet is attached below with depreciated current market value as per different category of the machines cumulated together. From the cumulative inventory sheet provided by the company, our engineering team has separated Plant & Machinery details, Auxiliary common machines for the complete plant and Main Plant Buildings.

Table: 13

| PART H | CONSOLIDATED VALUATION ASSESSMENT OF ASSETS OF THE PROJECT |
| --- | --- |

|  | Description | Value by adopting |  |  |
| --- | --- | --- | --- | --- |
|  | Valuation of the Property | Cost of Capitalization (Rs.) | Estimated Reproduction Cost of the Assets (Rs.) | Depreciated Replacement Fair Market Value (Rs.) |
|  | Land (A) | Rs.236,45,81,057/- | Rs.419,12,69,482/- | Rs.419,12,67,785/- |
|  | Buildings & Civil Works (B) | Rs.377,42,19,611/- | Rs.385,40,34,396/- | Rs.224,03,30,231/- |
|  | Plant & Machinery, Other Fixed Assets & Spares (C) | Rs.3424,16,42,185/- | Rs.3155,46,96,122/- | Rs.1605,73,17,921/- |
|  | Depreciated Asset Replacement Value Total {D= Add (A+B+C)} | Rs.4038,04,42,853/- | Rs.3960,00,00,000/- | Rs.2248,89,15,937/- |
|  | INR/ MW in Cr. | Rs.6.73 Cr./ MW | Rs.6.60 Cr./ MW | Rs.3.75 Cr./ MW |
|  | Consolidated Fixed Asset Valuation (E) |  | Rs.2248,89,15,937/- |  |
|  | Rounded Off (Depreciated Replacement Fair Market Value) |  | Rs.2249,00,00,000/- (Rs.3.75 Cr./MW) |  |
|  | Total Realizable/ Fetch Value of the Plant (Depreciated Replacement Fair Market Value-20%) |  | Rs.1799,20,00,000/- (Rs.3.00 Cr./MW) |  |
|  | Forced/ Distress Sale Value (Depreciated Replacement Fair Market Value-30%) |  | Rs.1574,30,00,000/- (Rs.2.62 Cr./MW) |  |
|  | Liquidation Value (Orderly Liquidation Value of the assets on Slump Sale) (Depreciated Replacement Fair Market Value-45%) |  | Rs.1236,95,00,000/- (Rs.2.06 Cr./MW) |  |

(Rupees Two Thousand Two Hundred Forty Nine Crore Only)

|  | Concluding comments if any | As per the scope of the assignment, Value assessment is subject to R.K Associates Important Notes and Valuer’s Remarks (Enclosure: 1) & other enclosed documents with the Report which will remain part & parcel of the report. |
| --- | --- | --- |

REMARKS:

- Valuation of the asset is done as found on as-is-where basis and primarily based on the Far provided to us.

- Consolidated Fair Market Fixed Asset Valuation comes out to be Rs.3.75 Cr. / MW which is well within CERC/ Industry benchmark of recently setup sub-critical thermal power plants in last few years which comes out to be around Rs.6.5- 7.0 Cr. / MW.

- All the Values includes soft cost incurred during the Project inception such as Pre-Operative expenses, Finance cost, IDC, etc. since the FAR is capitalized with all these soft cost and FAR was the main reference point for this Valuation assessment. For reaching out to the core Asset Valuation excluding Finance & IDC cost, Finance & IDC cost ratio taken in the FAR has to be excluded.

- For the calculation of Realizable value of Plant, we have taken various factors of the plant in to consideration:

- The subject project is based on Sub- Critical Technology having specification of 2x300 MW.

- The subject plant has no Power Purchase Agreement (PPA) and absence of FSA.

- Independent Power Projects in private sector are revenue generating assets and are primarily build to earn profits. If the asset built is unable to generate enough cash flows to meet its expenses and generate profits then it’s worth and value erodes. However since the core economic factor of the technologically viable asset is more related to income approach and therefore it may not affect its replacement value but this factor will affect the overall Fair Value or Fair Market Value of the asset. Therefore, considering all the factors, we have taken around 25% as economic obsolescence factor to arrive the Fair Value of the project. The obsolescence percentage has been considered based on the market trend and similar assets transactions done in the past in the market.

- The Realizable Value of the project has been taken considering the overall demand and interest towards power assets in the open market especially thermal power plant is currently very low due to overall low Industry utilization factor. Post COVID-19 situation the demand & outlook has plummeted additionally. Therefore this will affect the realisability of the asset.

- All the Values includes soft cost incurred during the Project inception such as Pre-Operative expenses, Finance cost, IDC, etc. since the FAR is capitalized with all these soft cost and FAR was the main reference point for this Valuation assessment. For reaching out to the core Asset Valuation excluding Finance & IDC cost, Finance & IDC cost ratio taken in the FAR has to be excluded.

- This report only contains general assessment & opinion on the Depreciated market value of the assets of the project found on as-is-where basis on site for which the Bank/ customer has shown & asked us to conduct the Valuation for which photographs is also attached with the report. No legal aspects in terms of ownership or any other legal aspect is taken into consideration. It doesn’t contain any due-diligence other than the valuation assessment of the property shown to us on site. Information/ data/ documents given to us by Bank/ client has been relied upon in good faith. This report doesn’t contain any other recommendations of any sort.

- This is just core Asset Valuation and doesn’t cover any prospective sale value of the Power Plant as a whole.

- Any kind of unpaid statutory, utilities, lease, interest or any other pecuniary dues on the asset has not been factored in the Valuation.

- This Valuation is conducted based on the macro analysis of the asset/ property considering it in totality and not based on the micro, component or item wise analysis. Analysis done is a general assessment and is not investigative in nature.

DEFINITIONS

- Fair Market Value suggested by the competent Valuer is that prospective estimated amount in his expert & prudent opinion of the subject asset/ property without any prejudice after he has carefully & exhaustively evaluated the facts & information came in front of him related to the subject asset at which the subject asset/ property should be exchanged between a willing buyer and willing seller at an arm’s length transaction in an open & unrestricted market, after proper marketing, wherein the parties, each acted knowledgeably, prudently and without any compulsion on the date of the Valuation.

Forced, under compulsion & constraint, obligatory sales transactions data doesn’t indicate the Fair Market Value.

- Realizable Value is the minimum prospective estimated value of the property which it may be able to realize at the time of actual property transaction factoring in the potential prospects of deep negotiations carried out between the buyer & seller for ultimately finalizing the transaction across the table. Realizable value may be 10-30% less than the Fair Market Value depending on the various salability prospects of the subject property and the needs of the buyer & the seller.

- Forced/ Distress Sale Value is the value when the property has to be sold due to any compulsion or constraint like financial encumbrances, dispute, as a part of a recovery process, any defect in the property, legal issues or any such condition or situation. In this type of sale, minimum fetch value is assessed which can be 25-40% less than the estimated Fair Market Value based on the nature, size & salability prospects of the property. In this type of sale, negotiation power of the buyer is always more than the seller and eagerness & pressure of selling the property is more than buying it. Therefore, the Forced/ Distress Sale Value will always fetch significantly less value compare to the estimated Fair Market Value.

- Liquidation Value As per International Valuation Standard Council, Liquidation Value is the amount that would be realized when an asset or group of assets are sold on a piecemeal basis that is without consideration of benefits (or detriments) associated with a going-concern business. Liquidation value can be either in an orderly transaction with a typical marketing period or in a forced transaction with a shortened marketing period.

In light of the Indian statutory context, Liquidation Value of the Physical Assets on ongoing concern basis can be best defined which is assigned to any asset as a whole in operational condition having some functional & utility value of that company/ industry/ business which is not doing good economically/ financially and is unable to repay its liabilities/ debts and has crossed all the stages of revival, restructuring & resolution available to it by law and is now in the last stage of revival by way of selling the assets by way of slump sale or which may generate interest in the buyer to take up the assets of the company in its last stage which has to be disposed of in a time bound manner.

In this condition the negotiation power of the buyer will be highest in this condition to buy the complete asset as a whole as it is and the discounting factor assigned is considering this market scenario & trend.

      - JUSTIFICATION FOR LIQUIDATION VALUE

- Liquidation value in this report is calculated based on the corporate debtor as a Standalone Basis assuming the mode of sale through private auction as per IBBI (Liquidation Process) Regulations, 2016 clause 35. In such a case Liquidation value on Slump Sale basis should be treated as the minimum reserve realizable Value.

- The premise of the liquidation value assessed in this report is only based on the CIRP Process as defined under IBC, 2016.

- As defined in the IBBI, LIQUIDATION Regulations, 2016 Liquidation value is the estimated realizable value of the assets of the corporate debtor if the corporate debtor were to be liquidated on the insolvency commencement date. However as per IBBI, Liquidation Regulation, 2016, the Liquidation process starts only after completion of LIQUIDATION which has the maximum deadline of 330 days.

- Liquidation value of the physical asset will not cover the Enterprise/ Business Value which can be done only through Enterprise/ Business Valuation principles using Income approach methodologies which will comprise of business future revenue projections, profitability aspects and business & financial risk associated with it.

- Liquidation value of the physical asset of the business on on-going concern basis will be basically the utility & operational value of the assets which any prudent buyer will be acquiring with the business to keep running the business without any interruption.

- In the present economic condition prevailing in the country, the demand for industrial properties in general and more so in thermal power sector is very less. However, some of the large power companies and ARCs have shown their interest in taking over distressed thermal power assets at highly discounted values.

- Due to emergence of renewable energy like solar and wind power in a big way and requirement of India for significantly lowering Carbon Emission Levels as per international commitments, interest in setting up new thermal power plants has faded away. Also due to disinterest and inability of Power Discoms in purchasing adequate power due to financial losses and Power theft is also keeping acting as a significant restraint in the ability of the power plants to sell power produced by them. However, the scenario is changing very gradually due to use of technology by Discoms to check power theft and hopefully the situation in this respect may get better in coming days.

- The liquidation value of the subject plant has been calculated on the basis of Slump Sale basis on which the plant can fetch its maximum value under Liquidation.

- Since the subject plant was in running condition till Janunary-2019 and after that company is doing maintenance of all the machinery timely, so all the machinery which was available at the site during the survey was in average condition and can be used again. And only a full scale overhauling needed to restart again.

- Therefore taking consideration all these factors the subject plant can fetch its maximum Liquidation Value (on slum sale concern) by weighing the Plant & Machinery as a whole.

- Recently, many transactions have taken place in power plant by “National Company Law Appellate Tribunal” out of those some are mentioned in the report.

- Difference between Costs, Price & Value: Generally, these words are used and understood synonymously. However, in reality each of these has a completely different meaning, premise and also have different definitions in the professional & legal terms. Therefore, to avoid confusion, it is our professional responsibility to describe the definitions of these words to avoid ambiguity & confusion in the minds of the user of this report.

  - The Cost of an asset represents the actual amount spend in the construction/ actual creation of the asset.

  - The Price is the amount paid for the procurement of the same asset.

  - The Value is defined as the present worth of future rights in the asset and depends to a great extent on combination of various factors such as demand and supply, market situation, purpose, situation & needs of the buyer & seller, saleability outlook, usability factor, market perception & reputation of the buyer & seller, saleability outlook, usability factor, market perception & reputation.

  - Therefore, in actual for the same asset, cost, price & value remain different since these terms have different usage & meaning.

REMARKS

- No employee or member of R.K Associates has any direct/ indirect interest in the Project.

- This is just an opinion report on Valuation based on the copy of the documents/ information provided to us by the client which has been relied upon in good faith and the assessment and assumptions done by us.

- This report is prepared based on the copies of the documents/ information which the Bank/ Company has provided to us out of the standard checklist of documents sought from them and further based on our assumptions and limiting conditions. The client/owner and its management/representatives warranted to us that the information they supplied was complete, accurate and true and correct to the best of their knowledge. All such information provided to us has been relied upon in good faith and we have assumed that it is true and correct in all respect. I/We shall not be liable for any loss, damages, cost or expenses arising from fraudulent acts, misrepresentations, or wilful default on part of the owner, company, its directors, employee, representative or agents. Verification or cross checking of the documents provided to us from the originals or from any Govt. departments/ Record of Registrar has not been done at our end since this is beyond the scope of our work. If at any time in future, it is found or came to our knowledge that misrepresentation of facts or incomplete or distorted information has been provided to us then this report shall automatically become null & void.

- This Valuation is prepared based on the current financial status of the company, financial data, other facts & information provided by the client in writing & during verbal discussion during the course of the assignment and based on certain assumptions which are specifically mentioned in the Valuation section of the Report.

- Key assumptions in the report is taken based on data, information, inputs, financial statements etc. provided by the client to us during the course of the assessment and on the basis of the assessment done by us and we have assumed that all such information is true & factual to the best of the knowledge of the promoter company.

- Sale transaction method of the asset is assumed as free market transaction while assessing Prospective Fair Market Value of the asset.

- Legal aspects for eg. investigation of title, ownership rights, lien, charge, mortgage, lease, sanctioned maps, verification of documents, etc. have not been done at our end and same has to be taken care by legal expert/ Advocate. It is assumed that the concerned Lender/ Financial Institution has satisfied them with the authenticity of the documents, information given to us and for which the legal verification has been already taken and cleared by the competent Advocate before requesting for this report. I/ We assume no responsibility for the legal matters including, but not limited to, legal or title concerns.

- All observations mentioned in the report is only based on the visual observation and the documents/ data/ information provided by the client. No mechanical/ technical tests, measurements or any design review have been performed or carried out from our side during Project assessment.

- Bank/FII should ONLY take this report as an Advisory document from the Financial/ Chartered Engineering firm and its specifically advised to the creditor to cross verify the original documents for the facts mentioned in the report which can be availed from the borrowing company directly.

- In case of any default in loans or the credit facility extended to the borrowing company, R.K Associates shall not be held responsible for whatsoever reason may be and any request for seeking any explanation from the employee/s of R.K Associates will not be entertained at any instance or situation.

- We have relied on data from third party, external sources & information available on public domain also to conclude this report. These sources are believed to be reliable and therefore, we assume no liability for the truth or accuracy of any data, opinions or estimates furnished by others that have been used in this analysis. Where we have relied on data, opinions or estimates from external sources, reasonable care has been taken to ensure that such data has been correctly extracted from those sources and /or reproduced in its proper form and context, however still we can’t vouch its authenticity, correctness or accuracy.

- This Report is prepared by our competent technical team which includes financial experts & analysts.

- Value varies with the Purpose/ Date/ Condition of the market. This report should not to be referred if any of these points are different from the one mentioned aforesaid in the Report.

- Analysis and conclusions adopted in the report are limited to the reported assumptions, conditions and information came to our knowledge during the course of the work.

- This report is having limited scope as per its fields to provide only the general indication of the Value of Equity of the companies prevailing in the market based on the documents/ data/ information/ financial statements provided by the client and the assessment and assumption taken by us. The suggested value should be considered only if transaction is happened as free market transaction.

- Secondary/ Tertiary costs related to transaction like Stamp Duty, Registration charges, Brokerage, etc. pertaining to the sale/ purchase of the company are not considered while assessing the Market Value.

- Appropriate methodology & assumptions are rationally adopted based on the facts of the case came to our knowledge during the course of the assignment considering many factors like nature of Industry, current market situation and trends.

- Valuation is a subjective field and opinion may differ from consultant to consultant. To check the right opinion, it is important to evaluate the methodology adopted and various factors/ basis considered during the course of assessment before reaching to any conclusion.

- At the outset, it is to be noted that Value analysis cannot be regarded as an exact science and the conclusions arrived at in many cases will, of necessity, be subjective and dependent on the exercise of individual judgment. Given the same set of facts and using the same assumptions, expert opinions may differ due to the number of different factors, which have to be made. Therefore, there can be no standard formulae to establish an indisputable exchange ratio. In the event of a transaction, the actual transaction value achieved may be higher or lower than our indicative analysis of value depending upon the circumstances of the transaction. The knowledge, negotiability and motivations of the buyers and sellers and the applicability of a discount or premium for control will also affect actual price achieved. Accordingly, our indicative analysis of value will not necessarily be the price at which any agreement proceeds. The final transaction price is something on which the parties themselves have to agree. However our Valuation analysis can definitely help the stakeholders to make them informed and wise decision about the Value of the asset and can help in facilitating the arm’s length transaction.

- This is just an opinion report and doesn’t hold any binding on anyone. It is requested from the concerned stakeholder which is using this report that they should consider all the different associated relevant & related factors also before taking any business decision based on the content of this report.

- Though adequate care has been taken while preparing this report as per its scope, but still we can’t rule out typing, human errors, over sightedness of any information or any other mistakes. Therefore, the concerned organization is advised to satisfy themselves that the report is complete & satisfactory in all respect. Intimation regarding any discrepancy shall be brought into our notice immediately. If no intimation is received within 15 (Fifteen) days in writing from the date of issuance of the report, to rectify these timely, then it shall be considered that the report is complete in all respect and has been accepted by the client upto their satisfaction & use and further to which R.K Associates shall not be held responsible in any manner.

- This Valuation report is prepared based on the facts of the companies provided to us during the course of the assignment. However, in future the assumptions taken may change or may go worse due to impact of Govt. policies or effect of World economy, Industry/ market scenario may change, etc. Hence before taking any business decision the user of this report should take into consideration all such future risk.

- The documents, information, data provided to us during the course of this assessment by the client is reviewed only upto the extent required in relation to the scope of the work. No document has been reviewed beyond the scope of the work.

- In case of any default in repayment of credit facility extended to the borrowing company, as estimated by us, R.K Associates shall not be held responsible for whatsoever reason may be and any request for seeking any explanation from the employee/s of R.K Associates will not be entertained at any instance or situation.

- This report only contains general assessment & opinion as per the scope of work evaluated and based on technical & market information which came to knowledge during course of the assignment as per the information given in the copy of documents, information, data provided to us and/ and confirmed by the owner/ owner representative to us at site which has been relied upon in good faith. It doesn’t contain any other recommendations of any sort including but not limited to express of any opinion on the suitability or otherwise of entering into any transaction with the borrower.

- This report is prepared following our Standard Operating Procedures & Best Practices, Limitations, Conditions, Remarks, Important Notes, Valuation TOR.

- Valuation is done based on the Industry wide general accepted norms and based on the international standards & best practices for equity valuations.

- Any kind of unpaid statutory, utilities, lease, interest or any other pecuniary dues on the asset has not been factored in the Valuation.

- This Valuation is conducted based on the macro analysis of the asset/ property and operations of the companies and not based on the micro, component or item wise analysis. Analysis done is a general assessment and is not investigative in nature.

- All Pages of the report including annexures are signed and stamped from our office. In case any paper in the report is without stamp & signature then this should not be considered a valid paper issued from this office.

- Defect Liability Period is 15 DAYS. We request the concerned authorized reader of this report to check the contents, data and calculations in the report within this period and intimate us in writing if any corrections are required or in case of any other concern with the contents or opinion mentioned in the report. Corrections only related to typographical, calculation, spelling mistakes, incorrect data/ figures/ statement will be entertained within the defect liability period. Any new changes for any additional information in already approved report will be regarded as additional work for which additional fees may be charged. No request for any illegitimate change in regard to any facts & figures will be entertained.

- R.K Associates encourages its customers to give feedback or inform concerns over its services through proper channel at valuers@rkassociates.org in writing within 15 days of report delivery. After this period no concern/ complaint/ proceedings in connection with the Financial Feasibility Study Services will be entertained due to possible change in situation and condition of the subject Project.

- Our Data retention policy is of ONE YEAR. After this period we remove all the concerned records related to the assignment from our repository. No clarification or query can be answered after this period due to unavailability of the data.

- This Fixed Assets Valuation report is governed by our (1) Internal Policies, Processes & Standard Operating Procedures, (2) R.K Associates Quality Policy, (3) Valuation & Survey Best Practices Guidelines formulated by management of R.K Associates, (4) Information input given to us by the customer and (4) Information/ Data/ Facts given to us by our field/ office technical team. Management of R.K Associates never gives acceptance to any unethical or unprofessional practice which may affect fair, correct & impartial assessment and which is against any prevailing law. In case of any indication of any negligence, default, incorrect, misleading, misrepresentation or distortion of facts in the report then it is the responsibility of the user of this report to immediately or at least within the defect liability period bring all such act into notice of R.K Associates management so that corrective measures can be taken instantly.

- R.K Associates never releases any report doing alterations or modifications from pen. In case any information/ figure of this report is found altered with pen then this report will automatically become null & void.

- If this report is prepared for the matter under litigation in any Indian court, no official or employee of R.K Associates will be under any obligation to give in person appearance in the court as a testimony. For any explanation or clarification, only written reply can be submitted on the additional payment of charges.

| Declaration | - The property was inspected by our authorized surveyor on 15 December 2021 by Mr. Hemant Ambaselkar, in the presence of the Company representative. - The undersigned does not have any direct/indirect interest in the above property. - The information furnished herein is true and correct to the best of our knowledge. - This valuation work is carried out by our Engineering team on the request from Axis Bank Limited, Strategic Solutions Group, Worli, Mumbai. - We have submitted Valuation report directly to the Bank. |  |  |
| --- | --- | --- | --- |
| Name & Address of Valuer company | Wealth Tax Registration No. |  | Signature of the authorized person |
| M/s R.K. Associates Valuers & Techno Engineering Consultants Pvt. Ltd. D-39,2nd Floor, Sector-02, Noida-201301 | 2303/ 1988 |  |  |
| Enclosed Documents | - Valuer’s Remark - Page No.51 - Rajasthan Govt. Circle Rate – Page No. 52-53 - Valuation of Building & Civil structures- 03 Pages - Valuation of Plant & machinery- 107 Pages - Photographs – 19 Pages |  |  |
| Number of Pages in the Report | Without Enclosures | 72 |  |
| Engineering Team worked on the report | SURVEYED BY: Mr. Hemant Ambaselkar |  |  |
|  | PREPARED BY: Project Engineering Team |  |  |
|  | REVIEWED BY: HOD Valuations |  |  |

For R.K Associates Valuers & Techno	Place	:	Delhi

Engineering Consultants (P) Ltd.	Date	:	16 December 2021

(Authorized Signatory)

Valuations

| PART I | RECENT DEALS IN POWER INDUSTRY |
| --- | --- |

Table: 16

| Sr. No. | Asset Description | Total Plant Capacity | Buyer | Value of Sale | INR (Cr.)/ MW |
| --- | --- | --- | --- | --- | --- |
|  | Sai Wardha | 540 MW | (One time Sttelment) | Rs.635 Cr | Rs.1.18 Cr/MW |
|  | Bhushan Energy | 456 Cr | (One time St telment) | Rs.730 Cr | Rs.1.57 Cr/MW |
|  | Korba West | 600 MW | Insolvency | Rs.1,167 Cr | Rs.1.95 Cr/MW |
|  | Essar Mahaan | 1200 MW | NCLT | Rs.2,800 Cr | Rs.2.4 Cr/MW |
|  | Lanco Vidarbha | 1320 MW | Liquidation | Rs.496 Cr | Rs.0.38 Cr/MW |
|  | Monnet Power | 1050 MW | Liquidation | Rs.630 Cr | Rs.0.60 Cr/MW |
|  | Lanco Babandh | 1320 MW | Under Liquidation-in progress | Rs.815 Cr | Rs.0.62 Cr/MW |

Notes:

1. This information is based in printed media sources and Industry publications.

2. Values reflected here are sale transactional values which are different from the Fixed Asset Valuation due to inherent limitations of different methodologies used.

3. This information is just for the informational purpose for the concerned organizations.

| PART J | ASSUMPTIONS/REMARKS |
| --- | --- |

|  | Information of the average market rates is taken based on the verbal market survey in the subject area from the local people, property agents, recent deals, demand-supply, internet postings which has been relied upon. No written record is generally available for such market information. |
| --- | --- |
|  | Sale transaction method of the asset is assumed as free market transaction while assessing Prospective Fair Market Value of the asset. |
|  | # All area measurements are on approximate basis. Verification of the area measurement of the property is done only based on sample random checking and not based on full scale measurement. Area of the large land parcels of more than 2500 sq.mtr or of uneven shape, is taken as per property documents which has been relied upon. |
|  | Legal aspects are not considered in this report. It is assumed and taken into account that the concerned Bank/ Financial Institution has first got the legal verification cleared by the competent Advocate before requesting for the Valuation report. |
|  | This report is prepared based on the copies of the documents/ information which interested organization or customer could provide to us out of the standard checklist of documents sought from them and further based on our assumptions and limiting conditions. All such information provided to us has been relied upon and we have assumed that it is true and correct. Verification or cross checking of the documents provided to us from the originals has not been done at our end. If at any time in future it’s found or came to our knowledge that misrepresentation of facts or incomplete or distorted information has been provided to us then this report will automatically become null & void. |
|  | Valuation is done for the property identified to us by the owner/ owner representative. Responsibility of identifying the correct property to the Valuer/ its authorized surveyor is solely of the client/ owner for which Valuation has to be carried out. At our end we can just cross verify the identification of the property with reference to the documents produced for perusal. Method by which identification of the property is carried out is also mentioned in the report clearly. It is requested from the Bank to cross check from their own records/ information if this is the same property for which Valuation has to be carried out to ensure that owner has not misled the Valuer company or misrepresented the property due to any vested interest. |
|  | In India more than 70% of the geographical area is lying under rural/ remote/ non municipal/ unplanned area where the subject property is surrounded by vacant lands having no physical demarcation or having any display of property survey or municipal number / name plate on the property clearly. Even in old areas of towns, small cities & districts where property number is not assigned clearly and not displayed on the properties, also due to the presence of multiple/ parallel departments (errors for in property registration) it becomes tough to identify the property with 100% surety from the available documents, information & site whereabouts and chances of error & misrepresentation by the borrower and margin & chances of error always persists in such cases. To avoid any such chances of error it is advised to the Bank to engage municipal/ revenue department officials to get the confirmation of the property to ensure that the property shown to Valuer/ Banker is the same as for which documents are provided. |
|  | If this Valuation Report is prepared for the Flat/ dwelling unit situated in a Group Housing Society or Integrated Township then it is assumed that the Banker or the concerned organization has satisfied themselves with the approval of the Group Housing Society/ particular floor & building before allotting the Valuation case to the Valuer company. |
|  | Due to fragmented & frequent change in building/ urban planning laws/ guidelines from time to time, different laws/ guidelines between regions/ states and no strict enforceability of Building Bye-Laws in India specially in non-metro and scale b & c cities & Industrial areas, property owners many times extend or make changes in the covered area/ layout out of approved/ applicable limits or the properties are decades old for which no formal Building Bye-Laws were applicable. Due to such discrete/ unplanned development in many regions sometimes it becomes tough to determine the exact lawful situation on ground for the Valuer. In case nothing specific is noted on the covered built-up area considered in the Valuation Report, the covered area present on the site as per site survey will be considered in the Valuation. |
|  | Valuation is a subjective field and opinion may differ from consultant to consultant. To check the right opinion, it is important to evaluate the methodology adopted and various factors/ basis considered during the course of assessment before reaching to any conclusion. Any kind of unpaid statutory, utilities, lease, interest or any other pecuniary dues on the asset has not been factored in the Valuation. |
|  | At the outset, it is to be noted that Value analysis of any asset cannot be regarded as an exact science and the conclusions arrived at in many cases will, of necessity, be subjective and dependent on the exercise of individual judgment. Given the same set of facts and using the same assumptions, expert opinions may differ due to the number of separate judgment decisions, which have to be made. Therefore, there can be no standard formulae to establish an indisputable exchange ratio. In the event of a transaction, the actual transaction value achieved may be higher or lower than our indicative analysis of value depending upon the circumstances of the transaction. The knowledge, negotiability and motivations of the buyers and sellers and the applicability of a discount or premium for control will also affect actual price achieved. Accordingly, our indicative analysis of value will not necessarily be the price at which any agreement proceeds. The final transaction price is something on which the parties themselves have to agree. However our Valuation analysis can definitely help the stakeholders to make them informed and wise decision about the Value of the asset and can help in facilitating the arm’s length transaction. |

| PART K | R.K ASSOCIATES IMPORTANT NOTES |
| --- | --- |

- As per the scope of the Report, Value assessment is subject to R.K Associates Important Notes and Valuer’s Remarks & other enclosed documents with the Report. Reader of the report is advised to read all the points mentioned in these sections carefully.

- If this report is used for Bank/ FIs then as per IBA Guidelines in case the valuation report submitted by the valuer is not in order, the banks / FIs shall bring the same to the notice of the valuer within 15 days of submission for rectification and resubmission. In case no such communication is received, it shall be presumed that the valuation report has been accepted.

- At our end we have not verified the authenticity of any documents provided to us. Bank is advised to verify the genuineness of the property documents before taking any credit decision.

- Information of the average market rates is taken based on the verbal market survey in the subject area from the local people, property agents, recent deals, demand-supply, internet postings. No written record is generally available for such market information.

- All area measurements are on approximate basis. Verification of the area measurement of the property is done only based on sample random checking and not based on full scale measurement.

- Valuation is done for the property identified to us by the owner/ owner representative. Method by which identification of the property is done is also mentioned in the report clearly. It is requested from the Bank to cross check from their own records/ information if this is the same property for which Valuation has to be carried out to ensure that owner has not misled the Valuer Company or misrepresented the property due to any vested interest.

- Legal aspects are not considered in this report. It is taken into account that the concerned Bank/ Financial Institution has first got the legal verification cleared by the competent Advocate before requesting for the Valuation report.

- Valuation is a subjective field and opinion may differ from consultant to consultant. To check the right opinion, it is important to evaluate the methodology adopted and various factors/ basis considered during the course of assessment before reaching to any conclusion.

- DEFECT LIABILITY PERIOD - In case of any query/ issue or escalation you may please contact Incident Manager by writing at valuers@rkassociates.org. We take all possible efforts to ensure 100% accuracy in the Calculations done, Rates adopted and various other data & information mentioned in the report but still can’t rule out typing, human errors or any other mistakes. In case you find any mistake, variation, discrepancy or inaccuracy in any data/ information of the report, please help us by bringing all such points into our notice in writing at valuers@rkassociates.orgwithin 30 days of the report delivery, to get these rectified timely, failing which R.K Associates Valuers Techno Engineering Consultants (P) Ltd. won’t be held responsible for any inaccuracy in any manner. Also if we will not hear back anything from you within 30 days, we will assume that report is correct in all respect and no further claim of any sort can be entertained thereafter. We would welcome and appreciate your feedback & suggestions in order to improve our services.

- COPYRIGHT FORMAT - This report is prepared on the copyright format of R.K Associates Valuers Techno Engineering Consultants (P) Ltd. to serve our clients with the best possible information. Legally no one can copy or distribute this format without prior approval from R.K Associates. It is meant only for the organization as mentioned on the cover page of this report and is strictly confidential. Distribution or use of this format other than R.K Associates will be seen as unlawful act and necessary legal action can be taken against the defaulters.

ENCLOSURE: 1– VALUER’S REMARKS

|  | Fair Market Value suggested by the competent Value in his opinion is an prospective estimated amount without any prejudice after evaluating all the facts related to the subject property at which the subject Asset should be exchanged on the date of Valuation between a willing buyer and willing seller at an arm’s length transaction after proper marketing, wherein the parties, each acted knowledgeably, prudently and without any compulsion. |
| --- | --- |
|  | Realizable Value is the minimum prospective value of the property which it may be able to realize at the time of actual property transaction factoring in potential prospects of deep negotiations carried out between the buyer & seller for ultimately finalizing the transaction. Realizable value may be 15-30% less of the Fair Market Value depending on the salability prospects of the subject property. |
|  | Forced/ Distress Sale Value is the value when the property has to be sold due to financial encumbrances or any other constraint or have become a disputed property or as a part of a recovery process. In this type of sale minimum disposable value is assessed which varies from 20-35% less from the Fair Market Value based on the salability prospects of the property. |
|  | Best rates are rationally adopted based on the facts of the case came to our knowledge during the course of the assignment considering many factors like nature of property, location, approach, market situation and trends. |
|  | Construction rates are adopted based on present replacement cost of construction and calculating applicable depreciation & deterioration factor as per its existing condition, specifications based on the visual observation of the structure. No physical tests have been carried out in respect of it. |
|  | No employee or member of R.K Associates has any direct/ indirect interest in the property. |
|  | Sale transaction method of the asset is assumed as free market transaction while assessing Fair Prospective Market Value of the asset. |
|  | Area measurements considered in the Valuation Report pertaining to Land & Building is adopted from relevant approved documents or actual site measurement whichever is less. All area measurements are on approximate basis only. |
|  | This report is having limited scope as per its fields to provide only the general basic idea of the value of the property prevailing in the market based on the documents/ data/ information provided by the client. The suggested value should be considered only if transaction is happened as free market transaction. |
|  | The condition assessment and the estimation of residual economic life of the structure is based on the visual observations and appearance found during the site survey. We have not carried out any structural design or stability study; nor carried out any physical tests to assess structural integrity & strength. |
|  | Secondary/ Tertiary costs related to asset transaction like Stamp Duty, Registration charges, Brokerage, etc. pertaining to the sale/ purchase of this property are not considered while assessing the Market Value. |
|  | This report is prepared based on the copies of the documents/ information which interested organization or customer could provide to us out of the standard checklist of documents sought from them and further based on our assumptions and limiting conditions. All such information provided to us has been relied upon and we have assumed that it is true and correct. Verification or cross checking of the documents provided to us from the originals has not been done at our end. If at any time in future it’s found or came to our knowledge that misrepresentation of facts or incomplete or distorted information has been provided to us then this report will automatically become null & void. |
|  | Investigation of title of the property and its legal right is beyond the scope of this report. If this property is offered as collateral security, the concerned financial institution is requested to verify & satisfy themselves on the ownership & legality of the property shown in this valuation report with respect to the latest legal opinion. |
|  | Value varies with the Purpose/ Date/ Condition of the market. This report should not to be referred if any of these points are different from the one mentioned aforesaid in the Report. |
|  | Analysis and conclusions adopted in the report are limited to the reported assumptions, conditions and information came to our knowledge during the course of the work. |
|  | This report is prepared on the RKA V-L5A (Very Large with P&M) Valuation format as per the client requirement, charges paid and the time allotted. This report is having limited scope as per its fields to provide only the general estimated basic idea of the value of the property prevailing in the market based on the information provided by the client. The Valuation assessed in this Valuation Report should hold good only if transaction is happened as per free market transaction. No detailed analysis or verification of the information is carried upon pertaining to the value of the subject property. No claim for any extra information will be entertained whatsoever be the reason. For any extra work over and above the fields mentioned in the report will have an extra cost which has to be borne by the customer. |
|  | This is just an opinion report and doesn’t hold any binding on anyone. It is requested from the concerned Financial Institution which is using this report for mortgaging the property that they should consider all the different associated relevant & related factors also before taking any business decision based on the content of this report. |
|  | This Valuation report is prepared based on the facts of the property on the date of the survey. However in future property Market may go down, property conditions may change or may go worse, Property reputation may differ, Property vicinity conditions may go down or become worse, Property market may change due to impact of Govt. policies or effect of World economy, Usability prospects of the property may change, etc. Hence before financing, Banker/ FI should take into consideration all such future risk and should loan conservatively to keep the advanced money safe in case of the downward trend of the property value. |
|  | All Pages of the report including annexure are signed and stamped from our office. In case any paper in the report is without stamp & signature then this should not be considered a valid paper issued from this office. |
|  | Defect Liability Period is 30 DAYS. We request the concerned authorized reader of this report to check the contents, data and calculations in the report within this period and intimate us in writing if any corrections are required or in case of any other concern with the contents or opinion mentioned in the report. Corrections only related to typographical, calculation, spelling mistakes will be entertained within the defect liability period. No request for any illegitimate value revision, date change or any other change will be entertained other than the one mentioned above. |
|  | R.K Associates encourages its customers to give feedback or inform concerns over its services through proper channel at valuers@rkassociates.org in writing within 30 days of report delivery. After this period no concern/ complaint/ proceedings in connection with the Valuation Services can be entertained due to possible change in situation and condition of the property. |
|  | Our Data retention policy is of ONE YEAR. After this period we remove all the concerned records related to the assignment from our repository. No clarification or query can be answered after this period due to unavailability of the data. |
|  | This Valuation report is governed by our (1) Internal Policies, Processes & Standard Operating Procedures, (2) R.K Associates Quality Policy, (3) Valuation & Survey Best Practices Guidelines formulated by management of R.K Associates, (4) Information input given to us by the customer and (4) Information/ Data/ Facts given to us by our field/ office technical team. Management of R.K Associates never gives acceptance to any unethical or unprofessional practice which may affect fair, correct & impartial assessment and which is against any prevailing law. In case of any indication of any negligence, default, incorrect, misleading, misrepresentation or distortion of facts in the report then it is the responsibility of the user of this report to immediately or atleast within the defect liability period bring all such act into notice of R.K Associates management so that corrective measures can be taken instantly. |
|  | R.K Associates never releases any report doing alterations or modifications from pen. In case any information/ figure of this report is found altered with pen then this report will automatically become null & void. |
|  | If this report is prepared for the matter under litigation in any Indian court, no official or employee of R.K Associates will be under any obligation to give in person appearance in the court as a testimony. For any explanation or clarification, only written reply can be submitted on payment of charges by the plaintiff or respondent which will be 10% of the original fees charged where minimum charges will be Rs.2500/-. |

ENCLOSURE: 2- MAHARASHTRA GOVERNMENT CIRCLE RATES

ENCLOSURE: 3 MIDC-BUTIBORI ALLOTMENT RATE