UTTAM SUGAR MILLS LTD. UNIT BARKATPUR (U.P)

Form I (1)

For Central Sugar Factories

Schedule of Machinery

Name of Factory          :  Uttam Sugar Mills Ltd. (Distillery Unit : - Barkatpur-Bijnor)

Capacity Licenced       :  52500 KL/ANNUM

75KLPD ETHANOL PLANT (MSDH)

| Sr. No. | Technical Data | Molecular Sieve Drum D-501A | Molecular Sieve Drum D-501B |
| --- | --- | --- | --- |
|  | Total Height of Column (mm) | 6000 | 6000 |
|  | Outer Diameter of Column (mm) | 1560 | 1560 |
|  | Inner Diameter of Column (mm) | 1550 | 1550 |
|  | Material of Construction | SS 304 | SS 304 |
|  | Thickness (mm) | 5 | 5 |
|  | Total no. of plates | NA | NA |
|  | Plate Spacing (mm) | NA | NA |
|  | Design details of Trays | NA | NA |
|  | Material Balance | - | - |
|  | Location & Design of feed plate | NA | NA |
|  | Capacity of column (LPH of spirit) | NA | NA |
|  | Percentage of alcohol in feed and output | - | - |
|  | Top and Base pressure of column (bar) | 1.76 / 1.80 | 1.76 / 1.80 |
|  | Pressure drop across tray mm WC (bar) | NA | NA |
|  | Vapour Velocity (Superficial velocity) m/sec | NA | NA |
|  | % of alcohol in reflux, reflux ratio | NA | NA |
|  | Efficiency | NA | NA |

75KLPD ETHANOL  PLANT  (MSDH)

|  | Units | Super Heater | Anhydrous Ethanol Condenser | Weak alcohol condenser | Weak alcohol Recirculation Cooler | Product Cooler |
| --- | --- | --- | --- | --- | --- | --- |
| 1. HTA | Sq.m | 72 | 65 | 65 | 60 | 80 |
| 2 2.Total number of the tubes | Nos. | 301 | 272 | 272 | 251 | 334 |
| 3.Length of the tubes | MM | 3000 | 3000 | 3000 | 3000 | 3000 |
| 4.Outer diameter of the tubes | MM | 25.4 | 25.4 | 25.4 | 25.4 | 25.4 |
| 5.Inner diameter of the tubes | MM | 23 | 23 | 23 | 23 | 23.0 |
| 6.Length of the shell | MM | 3750 | 3850 | 3750 | 3750 | 3750 |
| 7.Outer diameter of the shell | MM | 666 | 641 | 641 | 596 | 671 |
| 8.Inner diameter of the shell | MM | 660 | 635 | 635 | 590 | 665 |
| 9.Material of construction | -- | AISI 304 | AISI 304 | AISI 304 | AISI 304 | AISI 304 |
| 10.Shell Thickness | MM | 3 | 3 | 3 | 3 | 3 |
| 11. Tube Thickness | MM | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |
| 12.Tube Sheet Thickness | MM | 35 | 35 | 35 | 35 | 35 |
| 13.Heat transfer coefficient used | Kcal/hr.m2 0 C | 150 | 150 | 150 | 150 | 150 |
| 1 14.Temp. of the incoming coolant | 0 C | 32 | 32 | 32 | 32 | 32 |
| 15. Temp. of the outgoing coolant | 0 C | 40 | 40 | 40 | 40 | 40 |
| 16. Temp. of the incoming Product | 0 C | 105 | 130 | 130 | 130 | 130 |
| 17. Temp. of the outgoing product | 0 C | 105 | 130 | 130 | 130 | 35 |