REPORT FORMAT: CL-1 | Version: 1.0_2018

# FILE No.: VIS (2022-23)-PL186-141-289          	 	                     Date: 27/07/2022

- CERTIFICATE NAME: Capacity Assessment Certificate for Machineries situated at Property No. D-42, Industrial Estate, Selaqui, District Dehradun, Uttarakhand.

- PREPARED FOR: M/s. Theta Pharmaceuticals Limited

- ASSET TYPE: Pharmaceutical Industry related Machineries

- LOCATION OF THE BUILDING: Property No. D-42, Industrial Estate, Selaqui, District Dehradun, Uttarakhand.

TO WHOM IT MAY CONCERN

| S. NO. | PARTICULARS | DESCRIPTION |  |
| --- | --- | --- | --- |
|  | Name of the Certificate | Capacity Assessment of the Plant |  |
|  | Purpose of the Certificate | To ascertain the rated/ installed capacity of the Plant |  |
|  | Date of Survey | 19-07-2022 |  |
|  | Date of Certificate | 27-07-2022 |  |
|  | Documents pursued | Requested | Provided |
|  |  | List of Machines with make, model and specifications | List of Machines with make, model |
|  |  | Batch Manufacturing Record | Batch Manufacturing Record of Cetfast, Paracetamol, Cetirizine, Virgo, Calpro Tablet, Spurna Hair Oil |
|  |  | Invoices of Machines | None |
|  | Location of the Building | Property No. D-42, Industrial Estate, Selaqui, District Dehradun, Uttarakhand |  |
|  | Client Name | M/s. Theta Pharmaceuticals Limited |  |
|  | Type of Asset | Pharmaceutical Industry related machineries |  |
|  | Nature of Assets | General machinery used in pharmaceutical industry for the manufacturing of Tablets, Capsules, Ointment, Eye Drops & Oral Liquid. |  |
|  | Type of Assessment | Capacity Assessment |  |
|  | Scope of Assessment | To independently verify the rated/ installed capacity of the subject machines & verify the same with the capacity claimed by the company. |  |
|  | Out of Scope | - To estimate the present operational efficiency/ capacity utilization of the Plant. - Checking/ verifying any licences, approvals of the subject Industry. |  |
|  | Methodology/ Approach adopted | - Taking list of the machines to be verified with its specifications and capacity. - Site inspection of the machines noting Plate information of the machine. - Cross verifying the machine capacity of the same machine as available in the market. |  |

| PART A | CAPACITY ASSESSMENT ANALYSIS |
| --- | --- |

- LIST OF MACHINES VERIFIED AT SITE (SUBJECT MACHINES):

- CAPACITY ASSESSMENT OF SUBJECT MACHINES AS PER OUR MARKET RESEARCH V/S CAPACITY OF MACHINES CLAIMED BY COMPANY:

- SITE OBSERVATION:

  - The subject Plant is of manufacturing of different types of tablets & capsules.

  - All the machines during the time of site survey were found operational.

  - Most of the machine’s plates were showing its manufacturing date of 2007 model.

  - As per information given to us, M/s Theta Pharmaceuticals Limited has taken this manufacturing unit in IBC sale.

  - Main machine in the process is Compression Machine Station for manufacturing of Tablets based on whose capacity, Plant capacity is determined.

  - In Compression Machine Station granules are compressed to give a shape as per die with pressure to form the tablet.

  - Capsule filling machine is used to fill the drug granules in capsule.

- PROCEDURE ON CAPACITY ASSESSMENT:

- We have verified the subject machines as per the list provided to us by the company of which photographs are also attached.

- The company has provided us the capacity details of each machine and the same has been verified & compared with the similar machines available for sale on public domain.

- We have done the capacity assessment of the machines by comparing the capacity of machines claimed by the company from the capacity of similar machines available for sale on public domain.

- As per the information gathered on site & as per the discussion with the suppliers of such machines, we came to know that the actual production capacity of the machines depends upon various parameters like shape, size, weight of the product, flow of raw material etc.

- Fixed weight to capacity ratio or size to capacity ration is neither available from the supplier nor the company has any such detail.

- Rated Capacity claimed by the company / suppliers of the machines is the maximum capacity which a machine can attain under ideal condition. However, in practical and normal condition it may defer.

- We have also taken Batch Manufacturing Record of different products from the company on sample basis.

- On review of the records of Batch Manufacturing Record of some products on sample basis we found the following:

- As per our analysis, after studying the batch manufacturing record & discussion with the suppliers of similar machines we are of the view that actual production capacity of subject machines is different (lower than the installed capacity / maximum capacity).

- Therefore, as per the records perused and the information gathered from public domain as per our analysis for this Plant, the actual operational capacity can be considered as 15-20% less than the rated capacity of machines for the smallest tablet. It may further vary based on the size & weight of the tablet.

- Since there is a capacity variation due to size and weight of the tablet, therefore for the capacity assessment we have considered medium Cetfast tablet weighing 0.17 Gram as the base for this assessment. Therefore, based on the above assessment and factors we are of the opinion that this Plant comprising of the above machines is having atleast capacity to manufacture and package 10,75,067 tab/ hr per hour of small size having weight 0.17 Gram. This capacity will reduce if the size and weight of the tablet increases.

- The manufacturing and packaging capacity is also evaluated separately to check whether packaging capacity is complimenting the manufacturing capacity or not to complete the overall batch process. During which it is found that total rated manufacturing capacity of tablet + capsule is 12,76,667 tab/ hr (less of informed by the company vs claimed by supplier), whereas for packaging it is 10,75,067 tab/ hr (less of informed by the company vs claimed by supplier) which is approximately 2,01,600 less than the manufacturing capacity. Therefore, rated manufacturing capacity of the Plant can be considered as 10,75,067 tab/ per shift per day based on output capacity approach.

- Total actual assessed manufacturing capacity of tablet + capsule is 10,03,111 tab/ hr (considering batch production data) whereas for packaging it is 12,26,111 tab/ hr. In this case packaging capacity comes out to be more than the manufacturing capacity. Therefore, based on the above analysis we are of the opinion that for medium tablet of thickness equal to 2.99 mm and weight equal to 0.17 Gram, the manufacturing capacity of the Plant can be considered as 10,03,111 tab/ per shift per day. For variation in size and weight, the capacity will change.

- Other Section (Ophthalmic, Ointment, Dusting Powder) section machines are also evaluated as per the above procedure and accordingly after checking the machine capacity and the actual output capacity as per the batch record.

Disclaimer:

  - This Certificate is to be referred only for the purpose of the capacity assessment of the machineries situated at the aforesaid address.

  - This certificate doesn’t include any work related to drawing, design, sketch plan, procurement of the machines.

  - Ownership and other legal point of view in respect of the asset is not considered in this report as same is out of scope of this certificate.

  - This certificate is made at the request of the Client.

For R.K Associates Valuers & Techno 	 	 	        FOR INTERNAL USE        Engineering Consultants (P) Ltd.		 	 	TYPED BY:  SE Gaurav Sharma							                         REVIEWED BY: HOD Engineering

(Project Team)

# ANNEXURE: I- CAPACITY ASSESMENT WORKING

# ANNEXURE: II- LIST OF MACHINES

# ANNEXURE: III- BATCH MANUFACTURING RECORD

# ENCLOSURE: I- REFERNCES ON TECHNICAL SPECIFICATION OF SIMILAR MACHINES AVAILABLE ON PUBLIC DOMAIN

# ENCLOSURE: II- PHOTOGRAPHS