TITLE: MAINTENANCE SCHEDULES                ANNEXURE-1

| SNO | M/C | DESCRIPTION | PERIOD | REMARKS |
| --- | --- | --- | --- | --- |
| 1 | ## BLOWROOM | GENERAL CLEANING - VENTILATOR CLEANING - | 1 MONTH 3 MONTHS | +_10days +_1month |
| 2 |  | BEATER RECONDITIONING MBO ’ERM. UNIMIX. | NEED BASE |  |
| 3 |  | FAN CHANGE DEDUSTER & VENTILATOR | NEEDBASE |  |
| 4 |  | INCLINDED LATTICE MBO & UNIMIX | NEEDBASE |  |
| 5 | ## CARDING | GENERAL CLEANING | 15 DAYS | +_5days |
| 6 |  | FULL SETTING | 3 MONTHS | +_1months |
| 7 |  | GEAR BOX OIL CHANGE | 4 MONTHS | +_1.5months |
| 8 |  | LICKERIN WIRE CHANGE | 6 MONTHS | +_2months |
| 9 |  | DOFFER WIRE GRINDING | 6 MONTHS | +_2months |
| 10 |  | FLAT TOPS GRINDING | 6 MONTHS | +_3months |
| 11 |  | CYLINDER WIRE CHANGE | 24MONTHS | +_6months |
| 13 |  | DOFFER WIRE CHANGE | 24MONTHS | +_6months |
| 14 |  | FLAT TOPS CHANGE | 24MONTHS | +_6months |
| 15 |  | C-CLEANER SFL 14 & SFL24 CHANGE | 24MONTHS | +_6months |
| 16 |  | C-CLEANER SFD 62 CHANGE | 24MONTHS | +_6months |
| 17 |  | C-CLEANER POS CHANGE | 24M0NTHS | +_6months |
| 18 |  | FLAT CHAIN CHANGE | NEED BASE |  |
| 19 | ## LAP FORMER | GENERAL CLEANING | 15 DAYS | +_5days |
| 20 |  | GEAR BOX OIL CHANGE | 4 MONTHS | +_1.5months |
| 21 |  | COTS BUFFING | 1 MONTH | +_10days |
| 22 | ## COMBER | GENERAL CLEANING | 15 DAYS | +_5days |
| 23 |  | GEAR BOX OIL CHANGE | 4MONTHS | +_1.5months |
| 24 |  | CERCULAR.BRUSH.SETTING | 4MONTHS | +_1.5months |
| 25 |  | COTS BUFFING DRAW BOX | 1MONTHS | +_10days |
| 26 |  | FULL SETTING | 1 YEAR | +_3months |
| 27 |  | COTS BUFFING DETACHING | 2MONTHS | +_15days |
| 28 | ## D/F RSB-851 | GENERAL CLEANING | 15 DAYS | +_5days |
| 29 |  | GEAR BOX OIL CHANGE | 4MONTHS | +_1.5months |
| 30 |  | COTS BUFFING | 1MONTH | +_10days |
| 31 | ## D/F DO/ 6 | GENERAL CLEANING | 15 DAYS | +_5days |
| 32 |  | GEAR BOX OIL CHANGE | 6MONTHS | +_2months |
| 33 |  | COTS BUFFING | 1MONTHS | +_10days |
| 34 | ## SPEEDFRAME | GENEARAL CLEANING | 1MONTHS | +_10days |
| 35 |  | COTS BUFFING | 4MONTHS | +_1.5months |
| 36 |  | JACK BOX OIL CHANGE | 4MONTHS | +_1.5months |
| 37 |  | LONG COLLER CLEANING & FOOT STEP OILCHANGE | 4MONTHS | +_1.5months |
| 38 |  | BOTTOM ROLLER & SADDLE GAUGE | 1 YEAR | +_3months |
| 39 |  | BOBBIN RAIL LEVELING | 1 YEAR | +_3months |
| 40 |  | BOTTOM APRON CHANGE | 1.5YEARS | +_6months |
| 41 |  | TOP ROLLER ARBOUR GREASING | 1 YEAR | +_3months |
| 42 |  | TOP APRON CHANGE | 2YEARS | +_6months |