The Wind Turbine Generator

The E-53, 800 KW is suited for use both in central grid regions and in remote locations a long way from the main electricity grids. It owes this remarkable flexibility to the development of an intelligent WIND WORLD grid feeding system: state-of-the-art technology for today and tomorrow’s needs. Technical specifications of the 800 kW WTG are given in table 1 below.

Technical Details of E-53 (800 kW) WEG

| Particulars |  |
| --- | --- |
| Rated Capacity | 800 kW |
| Rotor Diameter | 53 m |
| Hub Height | 75 m |
| Rotor | With pitch control |
| Type | Upwind rotor with active pitch control |
| Direction of Rotation | Clockwise |
| Number of Blades | 3 |
| Swept Area | 2198 sq. m |
| Blade Material | Fiberglass (reinforced epoxy) with integral lighting protection |
| Rotor Speed | Variable, 12- 29 rpm |
| Pitch Control | 3 synchronized blade pitch system with emergency supply Generator with drive train |
| Hub | Rigid |
| Bearings | Tapered Roller bearings |
| Generator | Synchronous Type |
| Grid Feeding | Enercon Inverter |
| Braking System | 3 independent Aero Brakes |
| Yaw Control | Active through Adjustment Gears, Friction Damping |
| Cut-in Wind Speed | 2.5 m/s |
| Rated Wind Speed | 12 m/s |