Tempered Glass Heat Soak Furnace FY-JZ 3000/6000

Here is the tempered glass homogeneous plus laminating furnace’s actual parameter and the configuration. If you have further question, please call us anytime.

1. The Main Technical Standard：

  - Device Structure: The Furnace one side door is able to opened, 1 set of loading machine with wheel for 6 meter long glass loading

  - Device Specification：

Device outside dimensions： L6700×W2200×H4000 mm

Device inside dimensions：  L6400×W1200×H3800mm

Covers an area of： L15000×W4500×H5000mm

1.3 The Glass Production Specification：

Homogenous Glass The hugest size: 6000×3000 mm

Glass Thickness Range: 4-19mm

1.4 Product Performance: EN14179-1:2005

1.5 Total Power:329kW(Heating power288KW)

1.6 Voltage: 3 Cabins 415V /50Hz

2. The Main Technical Performance:

The tempering glass heat soak furnaces is made as the Europe standard which is strengthen safety of glasses. Adopting solid state relay. The computer can record and control the parameters intelligently. Temperature control mode is auto completed by electronic control system, which based on PLC and Kingview. The temperature and the result can be record and print automatically each time.There are six convection fans on the top , at the same time add control of cooling by 2 cooling fans. There are 20 thermocouples in the heat furnace body, can monitor temperature on surface of glasses from every area. Distribution of furnace top six thermocouple can monitor air temperature in the furnace

Technology parameters

1.Device outside dimensions: L6700×W2200×H4000 mm

2.Device inside dimensions:L6400×W1200×H3800 mm

3.The maxi size of glass :6000*3000mm

4.Thickness of glass: 4-19mm

5.The highest temperature:340℃

6.Working temperature: 290℃±10℃

7.Total power: 329KW

8.Heating power:288KW

9:6 sets of forced convection fan on the top( 5.5 kw/set)

10. two set of cooling fans( 4 kw/set), use cool water cooling system and air cooling cooling system.

11. 12 pcs of cold wind door, open and colse by air cylinder.

12.one side open of the furnace .

13. 1 set of 6 meter long A sheet, the bottom plate is weld for broken glass , .

14. Source: 0.5-0.7 MPA

15. Weight: 15T

16. Production capacity:8000 KG/Circle

Process parameters：

According to EN14179-1:2005 standards, it can achieve the following temperature and wind cycling by computer logic control

●Normal heating temperature(Below 50℃) to limit requirements temperature (290℃±10 ℃) 60 min-120 min;

●Keep the top limit temperature (290℃±10℃) 120 min-480 min;;

●Temperature to cooling temperature (70℃) 120 min--480 min; The maximum temperature is able to adjust

●±10℃ Empty furnace :the biggest difference temperature: ±10℃;

●±10℃ Full furnace temperature: the biggest temperature difference is :±10℃；

●Constant temperature: normal temperature to 320℃, can be set in a random；

●Constant temperature and time limit alarm temperature can be set in a random；

Attachment One

Heat soak time/temperature rules

To implementation of the equipment acceptance work to achieve EN14179-1:2005.The following requirements for process and control, as below:

1, calibration standard:

Homogeneous system must follow the picture 1. The system must be able to achieve the load of 100% and 10%.

Picture 1 Standard Homogeneous time/Temperature rules

T Any pints of glass temperature.

t Time hour

t1 Time of first piece of glass to 280℃

t2 Time of last piece of glass to 280℃

1 The first piece of glass temperature to 280℃

2 The last piece of glass temperature to 280℃

3 Glass temperature

a Heating period

b Constant temperature period

c Cooling period

d Ambient temperature

Note 1: full load depends on the size , the thickness of the glass and furnace volume. General base on 6 mm or 8 mm thickness, the distance between the glass depends on the location, type of glass and interal things shape decision.

Note 2: The minimum distance is 20 mm.

2. the collection of temperature

Inner furnace is no less than 6 points fixed temperature gathering and glass surface is more than 20 point temperature gathering, this some points is under the standards of the EN14179-1:2005 of installation, put, calibration .

Note 1: the furnace and glass surface temperature can be tested in full load .

Note 2: the furnace temperature can control unit monitoring at the air outlet. Glass surface temperature is measured of the hot good contact  thermocouple in the glass surface.

electrical control box

Mitsubishi PLC

PLC protection switch

Heating protection system

Ammeter

| Glass homogenizer (hot-dip) furnace FY-JZ6030main configuration list |  |  |  |
| --- | --- | --- | --- |
| Name | Specification | Qty | Brand |
| Furnace Steel pipe | 60*80 |  | Chinese Standard |
| Furnace door’s pipe | 50*100 |  | Chinese Standard |
| Steel pipes of the glass loading trolley | 80*80 40*40 |  | Chinese Standard |
| Furnace Cold Plate |  |  | Chinese Standard |
| Furnace Hot Plate |  |  | Chinese Standard |
| Furnace Interior | Stainless steel plate |  | Chinese Standard |
| Furnace duct interiors | Galvanized sheet |  | Chinese Standard |
| Punching guard outside the furnace | Cold Plate |  | Chinese Standard |
| Glass loading trolley rolling axis | 60mm diameter round |  | Chinese Standard |
| Convection Blower | 5.5Kw | 6sets | CHINA |
| Cooling Fan | 4Kw | 2setS | CHINA |
| PLC |  | 1 pcs | MITSUBISHI |
| PLC Expand |  | 1 pcs | MITSUBISHI |
| Frequency Converter |  | 1 pcs | VFD |
| Solid-State Relay |  | 36PCS | GOLD |
| Fuse |  | 36PCS | MRO |
| Computer |  | 1pcs | HP |
| monitor |  | 1pcs | DELL |
| Electric Appliance |  |  | Schneider+Delixi |
| Temperature acquisition module |  | 5pcs | Beijing |
| Temperature control software |  | 1set | Beijing kingview |
| Control Software |  |  | CHINA |
| Heating Tube |  | 72pcs | CHINA |
| Solenoid Valve |  |  | AirTAC |
| Track | 12 # rail steel |  | CHINA |
| Glass Spacer | Isolated glass width 20mm | Several | CHINA |

Machine description and Price

| No | Name of the product | Specification model | Qty (set) | Price (usd) | Amount (usd) | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Tempered Glass Heat Soak Furnace | FY-JZ6030 | 1 | 54500.00 | 54500.00 | The furnace body is divided into upper and lower sections for production |
| After-sales service and support within the warrantee period of 1 year. |  |  |  |  |  |  |
| Note: this quotation is FOB,1 40OT |  |  |  |  |  |  |

1、Payment Terms

40% -  as down payment upon order confirmation by T/T;

60% -  balance amount before delivery by T/T.

2、Delivery

Within 90 days from the receipt of down payment.

3、Using voltage is 415v/50Hz. Also it can be produced different voltage suitable for different voltage.

4、The buyer shall be responsible for the visa fee, round-trip transportation fee and accommodation fee of the installation engineer of the seller, and the installation and commissioning fee of the supplier shall be calculated at USD 100 / day.Quality assurance

5、From equipment installation date of acceptance by the buyer and seller from one year warranty.

6、After warranty period, supplier will charge the service cost.

FOSHAN FUYUAN GLASS Technology CO.,LTD

2025-2-11